import re

import numpy as np

import pandas as pd

import plotly.graph_objects as go

import plotly.offline as pyo

# ==============================================================================

# 1. INITIALIZE OFFLINE JUPYTER MODE (LOCAL ANACONDA ASSETS ONLY)

# ==============================================================================

pyo.init_notebook_mode(connected=False)

# ==============================================================================

# 2. STREAMED TERMINAL LOG DATA (Embedded In-Memory)

# ==============================================================================

raw_terminal_output = """

-System Online

>>> ACTIVITY: WALKING <<<<

STEP #1 | S | Dist: 0.5m | Pos: (0.1, -0.5)

[ENV] Pressure: 907.69 hPa

STEP #2 | 5 | Dist: 1.0m | Pos: (0.2, -1.0)

[ENV] Pressure: Err

STEP #3 | S | Dist: 1.5m | Pos: (0.4, -1.4)

[ENV] Pressure: 907.68 hPa

STEP #4 | 5 | Dist: 2.0m | Pos: (0.6, -1.9)

[ENV] Pressure: 907.58 hPa

STEP #5 |S| Dist: 2.5m | Pos: (0.7, -2.4)

[ENV] Pressure: 907.68 hPa

STEP #6|5| Dist: 3.0m | Pos: (0.7, -2.9)

[ENV] Pressure: 907.64 hPa

STEP #7 | S | Dist: 3.5m | Pos: (0.9, -3.4)

[ENV] Pressure: 907.67 hPa

STEP #8|5| Dist: 4.0m | Pos: (1.0, -3.9)

[ENV] Pressure: 907.63 hPa

STEP #9 | 5 | Dist: 4.5m | Pos: (1.1,-4.4)

[ENV] Pressure: Err

STEP #10 | SE | Dist: 5.0m | Pos: (1.3, -4.8)

[ENV] Pressure: 907.61 hPa

STEP #11 | SE | Dist: 5.5m | Pos: (1.5, -5.3)

[ENV] Pressure: 907.63 hPa

STEP #12 | S | Dist: 6.0m | Pos: (1.6, -5.8)

[ENV] Pressure: 907.66 hPa

STEP #13 | S | Dist: 6.5m | Pos: (1.8, -6.2)

[ENV] Pressure: 907.63 hPa

STEP #14 | S | Dist: 7.0m | Pos: (1.9, -6.7)

"""

# ==============================================================================

# 3. LOG RECTIFICATION & PARSING ENGINE

# ==============================================================================

def parse_terminal_log(text_data):

cardinal_map = {

"N": 0.0, "NORTH": 0.0,

"NE": 45.0, "NORTH-EAST": 45.0,

"E": 90.0, "EAST": 90.0,

"SE": 135.0, "SOUTH-EAST": 135.0,

"S": 180.0, "SOUTH": 180.0, "5": 180.0,  # Typo rectification '5' -> 'S'

"SW": 225.0, "SOUTH-WEST": 225.0,

"W": 270.0, "WEST": 270.0,

"NW": 315.0, "NORTH-WEST": 315.0

}

parsed_records = []

lines = text_data.strip().split("\n")

i = 0

last_valid_pressure = 907.69  # Baseline fallback

while i < len(lines):

line = lines[i].strip()

if "STEP #" in line:

step_num_match = re.search(r'STEP\s*#([0-9]+)', line, re.IGNORECASE)

step_num = int(step_num_match.group(1)) if step_num_match else len(parsed_records) + 1

dir_match = re.search(r'\|\s*([A-Za-z0-9]+)\s*\|', line)

raw_dir = dir_match.group(1).upper() if dir_match else "S"

if raw_dir == "5":

raw_dir = "S"

heading_deg = cardinal_map.get(raw_dir, 180.0)

dist_match = re.search(r'Dist:\s*([0-9.]+)m', line, re.IGNORECASE)

dist = float(dist_match.group(1)) if dist_match else 0.5

pos_match = re.search(r'Pos:\s*\(\s*([-0-9.]+)\s*,\s*([-0-9.]+)\s*\)', line)

x_val = float(pos_match.group(1)) if pos_match else None

y_val = float(pos_match.group(2)) if pos_match else None

pressure_val = last_valid_pressure

if (i + 1) < len(lines) and "[ENV]" in lines[i + 1]:

env_line = lines[i + 1].strip()

press_match = re.search(r'Pressure:\s*([0-9.]+)', env_line, re.IGNORECASE)

if press_match:

pressure_val = float(press_match.group(1))

last_valid_pressure = pressure_val

i += 1

parsed_records.append({

"step": step_num,

"direction": raw_dir,

"heading_deg": heading_deg,

"distance": dist,

"X_log": x_val,

"Y_log": y_val,

"pressure": pressure_val

})

i += 1

return pd.DataFrame(parsed_records)

df = parse_terminal_log(raw_terminal_output)

# ==============================================================================

# 4. OVERALL FLOOR TRANSITION DETECTION (START vs. END PRESSURE)

# ==============================================================================

p_start = df["pressure"].iloc[0]

p_end = df["pressure"].iloc[-1]

net_dp = p_end - p_start  # Net pressure difference in hPa

# Threshold: ~0.04 hPa (~4 Pa difference)

if net_dp < -0.04:

floor_transition = "⬆️ MOVING UPWARD (Ground to 1st Floor / Higher)"

elif net_dp > 0.04:

floor_transition = "⬇️ MOVING DOWNWARD (1st Floor to Ground / Lower)"

else:

floor_transition = "➡️ SAME FLOOR (Flat Ground Navigation)"

print(f"📊 Start Pressure : {p_start:.2f} hPa")

print(f"📊 End Pressure   : {p_end:.2f} hPa")

print(f"📊 Net Difference : {net_dp:.3f} hPa ({net_dp * 100:.1f} Pa)")

print(f"🧭 Overall Status : {floor_transition}")

# ==============================================================================

# 5. ELEVATION CALCULATION & HEAVY SMOOTHING FILTER

# ==============================================================================

# Raw Hypsometric Altitude Z (Meters)

df["Z_raw"] = 44330.0 * (1.0 - np.power((df["pressure"] / p_start), 0.1903))

# Enhanced Smoothing: Centered Rolling Average (Window = 5) for ultra-smooth trajectory

df["Z_meters"] = df["Z_raw"].rolling(window=5, min_periods=1, center=True).mean()

df["X_pos"] = df["X_log"]

df["Y_pos"] = df["Y_log"]

X_plot = np.insert(df["X_pos"].values, 0, 0.0)

Y_plot = np.insert(df["Y_pos"].values, 0, 0.0)

Z_plot = np.insert(df["Z_meters"].values, 0, 0.0)

# ==============================================================================

# 6. DYNAMIC 3D HOVER TOOLTIPS

# ==============================================================================

hover_texts = [

"<b>Origin Point (Start)</b><br>"

"X: 0.00m, Y: 0.00m, Height: 0.00m<br>"

f"Start Pressure: {p_start:.2f} hPa"

]

for i in range(len(df)):

is_last_step = (i == len(df) - 1)

if is_last_step:

text = (

f"<b>Step #{df.loc[i, 'step']} (Destination End)</b><br>"

f"<b>Overall Journey:</b> {floor_transition}<br>"

f"<b>End Pressure:</b> {df.loc[i, 'pressure']:.2f} hPa (Net ΔP: {net_dp:.2f} hPa)<br>"

f"<b>Final Elevation (Z):</b> {df.loc[i, 'Z_meters']:.2f} m<br>"

f"<b>Final Pos (X, Y):</b> ({df.loc[i, 'X_pos']:.2f}m, {df.loc[i, 'Y_pos']:.2f}m)<br>"

f"<b>Heading:</b> {df.loc[i, 'direction']} ({df.loc[i, 'heading_deg']:.0f}°)"

)

else:

text = (

f"<b>Step #{df.loc[i, 'step']}</b><br>"

f"<b>Pressure:</b> {df.loc[i, 'pressure']:.2f} hPa<br>"

f"<b>Smoothed Elevation (Z):</b> {df.loc[i, 'Z_meters']:.2f} m<br>"

f"<b>Pos (X, Y):</b> ({df.loc[i, 'X_pos']:.2f}m, {df.loc[i, 'Y_pos']:.2f}m)<br>"

f"<b>Heading:</b> {df.loc[i, 'direction']} ({df.loc[i, 'heading_deg']:.0f}°)<br>"

f"<b>Total Dist:</b> {df.loc[i, 'distance']:.1f} m"

)

hover_texts.append(text)

# ==============================================================================

# 7. INTERACTIVE 3D GRAPH GENERATION

# ==============================================================================

fig = go.Figure()

# 3D Trajectory Path Line

fig.add_trace(go.Scatter3d(

x=X_plot, y=Y_plot, z=Z_plot,

mode='lines',

line=dict(color='black', width=4),

name='Smoothed Path',

hoverinfo='skip'

))

# 3D Colored Step Nodes

fig.add_trace(go.Scatter3d(

x=X_plot, y=Y_plot, z=Z_plot,

mode='markers',

marker=dict(

size=7,

color=Z_plot,

colorscale='Viridis',

colorbar=dict(title="Elevation Z (m)"),

showscale=True,

line=dict(color='black', width=1)

),

text=hover_texts,

hoverinfo='text',

name='Steps'

))

# Start Point Marker (Green Diamond)

fig.add_trace(go.Scatter3d(

x=[X_plot[0]], y=[Y_plot[0]], z=[Z_plot[0]],

mode='markers',

marker=dict(size=12, color='green', symbol='diamond'),

text=[hover_texts[0]],

hoverinfo='text',

name='Start'

))

# Final Destination End Marker (Red Square)

fig.add_trace(go.Scatter3d(

x=[X_plot[-1]], y=[Y_plot[-1]], z=[Z_plot[-1]],

mode='markers',

marker=dict(size=12, color='red', symbol='square'),

text=[hover_texts[-1]],

hoverinfo='text',

name='Destination'

))

# 3D Scene Configuration

fig.update_layout(

title=f"<b>3D Indoor Navigation Path</b><br><sup>Transition: {floor_transition} | Net ΔP: {net_dp:.2f} hPa</sup>",

scene=dict(

xaxis=dict(title='X Position (Meters)', backgroundcolor="rgb(245, 245, 245)"),

yaxis=dict(title='Y Position (Meters)', backgroundcolor="rgb(245, 245, 245)"),

zaxis=dict(title='Height / Elevation Z (Meters)', backgroundcolor="rgb(230, 240, 250)"),

aspectmode='data'

),

margin=dict(l=0, r=0, b=0, t=60),

legend=dict(x=0.01, y=0.99)

)

# Render inside Jupyter Notebook Cell Output

pyo.iplot(fig)




2nd prgm


import re

import numpy as np

import pandas as pd

import plotly.graph_objects as go

import plotly.offline as pyo

# ==============================================================================

# 1. INITIALIZE OFFLINE JUPYTER MODE

# ==============================================================================

pyo.init_notebook_mode(connected=False)

# ==============================================================================

# 2. STREAMED TERMINAL LOG DATA

# ==============================================================================

raw_terminal_output = """

-System Online

>>> ACTIVITY: WALKING <<<<

STEP #1 | S | Dist: 0.5m | Pos: (0.1, -0.5)

[ENV] Pressure: 900.69 hPa

STEP #2 | 5 | Dist: 1.0m | Pos: (0.2, -1.0)

[ENV] Pressure: Err

STEP #3 | S | Dist: 1.5m | Pos: (0.4, -1.4)

[ENV] Pressure: 901.68 hPa

STEP #4 | 5 | Dist: 2.0m | Pos: (0.6, -1.9)

[ENV] Pressure: 902.58 hPa

STEP #5 |S| Dist: 2.5m | Pos: (0.7, -2.4)

[ENV] Pressure: 903.68 hPa

STEP #6|5| Dist: 3.0m | Pos: (0.7, -2.9)

[ENV] Pressure: 904.64 hPa

STEP #7 | S | Dist: 3.5m | Pos: (0.9, -3.4)

[ENV] Pressure: 905.67 hPa

STEP #8|5| Dist: 4.0m | Pos: (1.0, -3.9)

[ENV] Pressure: 906.63 hPa

STEP #9 | 5 | Dist: 4.5m | Pos: (1.1,-4.4)

[ENV] Pressure: Err

STEP #10 | SE | Dist: 5.0m | Pos: (1.3, -4.8)

[ENV] Pressure: 908.61 hPa

STEP #11 | SE | Dist: 5.5m | Pos: (1.5, -5.3)

[ENV] Pressure: 909.63 hPa

STEP #12 | S | Dist: 6.0m | Pos: (1.6, -5.8)

[ENV] Pressure: 910.66 hPa

STEP #13 | S | Dist: 6.5m | Pos: (1.8, -6.2)

[ENV] Pressure: 911.63 hPa

STEP #14 | S | Dist: 7.0m | Pos: (1.9, -6.7)

[ENV] Pressure: 912.67 hPa

"""

# ==============================================================================

# 3. LOG PARSING ENGINE

# ==============================================================================

def parse_terminal_log(text_data):

cardinal_map = {

"N": 0.0,

"NORTH": 0.0,

"NE": 45.0,

"NORTH-EAST": 45.0,

"E": 90.0,

"EAST": 90.0,

"SE": 135.0,

"SOUTH-EAST": 135.0,

"S": 180.0,

"SOUTH": 180.0,

"5": 180.0,       # Correct typo: 5 -> S

"SW": 225.0,

"SOUTH-WEST": 225.0,

"W": 270.0,

"WEST": 270.0,

"NW": 315.0,

"NORTH-WEST": 315.0

}

parsed_records = []

lines = text_data.strip().split("\n")

i = 0

# First valid pressure used only if the first pressure is Err

last_valid_pressure = None

while i < len(lines):

line = lines[i].strip()

# ----------------------------------------------------------------------

# Detect STEP line

# ----------------------------------------------------------------------

if "STEP #" in line:

step_match = re.search(

r"STEP\s*#(\d+)\s*\|\s*([A-Za-z0-9]+)\s*\|\s*Dist:\s*([\d.]+)m\s*\|\s*Pos:\s*\(([-\d.]+),\s*([-\d.]+)\)",

line

)

if step_match:

step_number = int(step_match.group(1))

direction_raw = step_match.group(2).upper()

distance = float(step_match.group(3))

x_position = float(step_match.group(4))

y_position = float(step_match.group(5))

# Correct "5" typo to "S"

direction = "S" if direction_raw == "5" else direction_raw

heading = cardinal_map.get(

direction_raw,

cardinal_map.get(direction, np.nan)

)

# --------------------------------------------------------------

# Read pressure from next line

# --------------------------------------------------------------

pressure = np.nan

if i + 1 < len(lines):

pressure_line = lines[i + 1].strip()

pressure_match = re.search(

r"Pressure:\s*([\d.]+|Err)",

pressure_line

)

if pressure_match:

pressure_value = pressure_match.group(1)

if pressure_value == "Err":

# Use previous valid pressure

if last_valid_pressure is not None:

pressure = last_valid_pressure

else:

pressure = float(pressure_value)

last_valid_pressure = pressure

# --------------------------------------------------------------

# Save record

# --------------------------------------------------------------

parsed_records.append({

"Step": step_number,

"Direction": direction,

"Heading": heading,

"Distance": distance,

"X": x_position,

"Y": y_position,

"Pressure": pressure

})

i += 1

return pd.DataFrame(parsed_records)

# ==============================================================================

# 4. PARSE DATA

# ==============================================================================

df = parse_terminal_log(raw_terminal_output)

# ==============================================================================

# 5. DISPLAY PARSED DATA

# ==============================================================================

print("PARSED TERMINAL DATA")

print("=" * 80)

print(df.to_string(index=False))

# ==============================================================================

# 6. PRESSURE VS DISTANCE GRAPH

# ==============================================================================

fig_pressure = go.Figure()

fig_pressure.add_trace(

go.Scatter(

x=df["Distance"],

y=df["Pressure"],

mode="lines+markers",

name="Pressure",

text=[

f"Step {step}<br>"

f"Direction: {direction}<br>"

f"Distance: {distance:.1f} m<br>"

f"Pressure: {pressure:.2f} hPa"

for step, direction, distance, pressure

in zip(

df["Step"],

df["Direction"],

df["Distance"],

df["Pressure"]

)

],

hoverinfo="text"

)

)

fig_pressure.update_layout(

title="Pressure vs Walking Distance",

xaxis_title="Distance (m)",

yaxis_title="Pressure (hPa)",

template="plotly_white"

)

pyo.iplot(fig_pressure)

# ==============================================================================

# 7. 2D WALKING PATH

# ==============================================================================

fig_path = go.Figure()

fig_path.add_trace(

go.Scatter(

x=df["X"],

y=df["Y"],

mode="lines+markers+text",

text=df["Step"].astype(str),

textposition="top center",

name="Walking Path"

)

)

fig_path.update_layout(

title="2D Walking Path",

xaxis_title="X Position (m)",

yaxis_title="Y Position (m)",

template="plotly_white",

xaxis=dict(scaleanchor="y")

)

pyo.iplot(fig_path)

# ==============================================================================

# 8. PRESSURE CHANGE PER STEP

# ==============================================================================

fig_step_pressure = go.Figure()

fig_step_pressure.add_trace(

go.Bar(

x=df["Step"],

y=df["Pressure"],

text=df["Pressure"].round(2),

textposition="outside",

name="Pressure"

)

)

fig_step_pressure.update_layout(

title="Pressure at Each Step",

xaxis_title="Step Number",

yaxis_title="Pressure (hPa)",

template="plotly_white"

)

pyo.iplot(fig_step_pressure)





3rd code