Here is a complete, offline-ready Python script using Matplotlib and Pandas that you can run directly inside a Jupyter Notebook.

It takes your manual input log containing Distance, Direction, and Pressure (hPa), converts them into Cartesian coordinates X,Y,Z, and generates an interactive, high-resolution 3D trajectory graph showing floor-to-floor indoor navigation.

How the Math Works Internally:

- X,Y Position Tracking:

  - Direction (N, NE, E, SE, S, SW, W, NW or Angles in degrees) is converted to radians.

  - Coordinate updates:

Xk=Xk-1+Distance×sinHeading

Yk=Yk-1+Distance×cosHeading

- Z (Altitude / Floor Level) Tracking:

  - Atmospheric pressure (PhPa) is converted into relative vertical altitude (Z in meters) using the hypsometric formula against your starting baseline pressure (P0):

Z=44330×1-PhPaP00.1903

  - Floor index is resolved assuming 1 Floor=3.0 meters.

Complete Python Script for Jupyter Notebook

Run this code in a Jupyter cell:

Python

import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

from mpl_toolkits.mplot3d import Axes3D

# Enable interactive 3D rotating graph in Jupyter (Optional)

# %matplotlib notebook  # Use this for classic Jupyter

# %matplotlib widget    # Use this for Jupyter Lab

# ==============================================================================

# 1. MANUAL INPUT DATASET

#    Format: Distance (meters), Direction (Compass string or Degrees), Pressure (hPa)

# ==============================================================================

manual_data = [

# Floor 0 Walk (Baseline ~1013.25 hPa)

{"step": 1,  "distance": 1.0, "direction": "N",  "pressure_hpa": 1013.25},

{"step": 2,  "distance": 1.0, "direction": "N",  "pressure_hpa": 1013.25},

{"step": 3,  "distance": 1.0, "direction": "E",  "pressure_hpa": 1013.24},

{"step": 4,  "distance": 1.0, "direction": "E",  "pressure_hpa": 1013.25},

# Climbing Staircase to Floor 1 (~3m climb -> Pressure drops by ~0.35 hPa)

{"step": 5,  "distance": 0.5, "direction": "E",  "pressure_hpa": 1013.15},

{"step": 6,  "distance": 0.5, "direction": "E",  "pressure_hpa": 1013.00},

{"step": 7,  "distance": 0.5, "direction": "S",  "pressure_hpa": 1012.90},

# Floor 1 Walk

{"step": 8,  "distance": 1.0, "direction": "S",  "pressure_hpa": 1012.89},

{"step": 9,  "distance": 1.0, "direction": "S",  "pressure_hpa": 1012.90},

{"step": 10, "distance": 1.0, "direction": "W",  "pressure_hpa": 1012.88},

{"step": 11, "distance": 1.0, "direction": "W",  "pressure_hpa": 1012.89},

# Climbing Staircase to Floor 2 (~6m total climb)

{"step": 12, "distance": 0.5, "direction": "W",  "pressure_hpa": 1012.70},

{"step": 13, "distance": 0.5, "direction": "N",  "pressure_hpa": 1012.55},

# Floor 2 Walk

{"step": 14, "distance": 1.0, "direction": "N",  "pressure_hpa": 1012.53},

{"step": 15, "distance": 1.0, "direction": "N",  "pressure_hpa": 1012.54},

]

# Convert manual list into Pandas DataFrame

df = pd.DataFrame(manual_data)

# ==============================================================================

# 2. DIRECTION CONVERSION & POSITION COMPUTATION (X, Y, Z)

# ==============================================================================

cardinal_map = {

"N": 0.0, "NE": 45.0, "E": 90.0, "SE": 135.0,

"S": 180.0, "SW": 225.0, "W": 270.0, "NW": 315.0

}

def get_heading_degrees(val):

if isinstance(val, str):

return cardinal_map.get(val.upper(), 0.0)

return float(val)

# Extract Baseline Pressure (Starting ground pressure)

P0 = df["pressure_hpa"].iloc[0]

# Compute Relative Height Z (Meters) and Floor Level

# Hypsometric Formula: Z = 44330 * (1 - (P / P0)^0.1903)

df["Z_meters"] = 44330.0 * (1.0 - np.pow((df["pressure_hpa"] / P0), 0.1903))

df["floor_index"] = np.round(df["Z_meters"] / 3.0).astype(int)

# Compute X and Y Coordinates

x_coords = [0.0]

y_coords = [0.0]

for i in range(len(df)):

dist = df.loc[i, "distance"]

heading_deg = get_heading_degrees(df.loc[i, "direction"])

rad = np.radians(heading_deg)

# Polar to Cartesian coordinate propagation

new_x = x_coords[-1] + dist * np.sin(rad)

new_y = y_coords[-1] + dist * np.cos(rad)

x_coords.append(new_x)

y_coords.append(new_y)

# Attach calculated positions to DataFrame (Including starting point at index 0)

df["X_pos"] = x_coords[1:]

df["Y_pos"] = y_coords[1:]

# Print calculated navigation table

print("=== CALCULATED INDOOR NAVIGATION LOG ===")

print(df[["step", "direction", "distance", "pressure_hpa", "X_pos", "Y_pos", "Z_meters", "floor_index"]])

# ==============================================================================

# 3. 3D PLOTTING (CLEAR FLOOR-TO-FLOOR NAVIGATION GRAPH)

# ==============================================================================

fig = plt.figure(figsize=(12, 9))

ax = fig.add_subplot(111, projection='3d')

# Full coordinate arrays including starting origin (0, 0, Z0)

X_plot = np.insert(df["X_pos"].values, 0, 0.0)

Y_plot = np.insert(df["Y_pos"].values, 0, 0.0)

Z_plot = np.insert(df["Z_meters"].values, 0, df["Z_meters"].iloc[0])

# Plot 3D Path trajectory colored by height

sc = ax.scatter(X_plot, Y_plot, Z_plot, c=Z_plot, cmap='coolwarm', s=60, edgecolors='k', zorder=5)

ax.plot(X_plot, Y_plot, Z_plot, color='black', linestyle='--', linewidth=2, alpha=0.7, label='Walk Path')

# Highlight Start (Green) and End (Red) Points

ax.scatter(X_plot[0], Y_plot[0], Z_plot[0], color='green', s=150, marker='^', label='Start (Origin)')

ax.scatter(X_plot[-1], Y_plot[-1], Z_plot[-1], color='red', s=150, marker='v', label='Current Position')

# Draw Discrete Floor Planes for Clear Visual Separation

min_x, max_x = min(X_plot) - 1, max(X_plot) + 1

min_y, max_y = min(Y_plot) - 1, max(Y_plot) + 1

xx, yy = np.meshgrid(np.linspace(min_x, max_x, 10), np.linspace(min_y, max_y, 10))

unique_floors = df["floor_index"].unique()

colors = ['gray', 'cyan', 'orange', 'purple']

for floor in unique_floors:

floor_z = floor * 3.0  # 3 meters per floor

color = colors[floor % len(colors)]

ax.plot_surface(xx, yy, np.full_like(xx, floor_z), alpha=0.15, color=color)

ax.text(min_x, max_y, floor_z + 0.2, f"  FLOOR {floor} ({floor_z:.1f}m)", color='darkblue', fontweight='bold')

# Annotate Step Numbers on the 3D plot

for i, txt in enumerate(range(len(X_plot))):

if txt == 0:

continue

ax.text(X_plot[i], Y_plot[i], Z_plot[i] + 0.1, f" S{txt}", fontsize=8)

# Formatting Graph Labels & Axes

ax.set_title("3D Indoor Pedestrian Dead Reckoning (Floor-to-Floor)", fontsize=14, fontweight='bold')

ax.set_xlabel("X Position (Meters)", fontweight='bold')

ax.set_ylabel("Y Position (Meters)", fontweight='bold')

ax.set_zlabel("Height Z / Altitude (Meters)", fontweight='bold')

# Colorbar for height reference

cbar = fig.colorbar(sc, ax=ax, shrink=0.5, aspect=10)

cbar.set_label('Elevation (Meters)', fontweight='bold')

ax.legend(loc='upper left')

plt.tight_layout()

plt.show()

Features of This Visualizer:

- Interactive Floor Planes: Automatically draws subtle horizontal planes at 0m (Floor 0), 3m (Floor 1), 6m (Floor 2), making vertical movement across floors immediately readable.

- Color-Coded Elevation: The trajectory line changes color smoothly from blue (Ground Level) to red (Upper Levels).

- Flexible Direction Input: Supports both cardinal text directions ("N", "NE", "E", "S", "W", etc.) and exact compass angles in degrees (90, 180, 270).

- Step Annotations: Automatically labels step points (S1, S2, S3) along the spatial path.

Here is the complete Python script that automatically extracts your telemetry logs directly from a text or log file, calculates the X,Y,Z coordinates, and plots the interactive 3D floor-to-floor navigation trajectory.

You can copy-paste your console output straight into a text file (e.g., sensor_output.txt), run this script in Jupyter Notebook or standard Python, and it will parse everything automatically.

Step 1: Save Your Sensor Output to a Text File

Create a file named sensor_output.txt in the same folder as your script and paste your console output inside it.

It handles multiple common log formats automatically (such as Press: 1013.25 hPa | Dist: 1.00m | Facing: EAST or step1 dire E dis 1m pressure 1013.25).

Example sensor_output.txt content:

Plaintext

🟢 [STEP #001] | Dist: 1.00m | Facing: NORTH | Press: 1013.25 hPa

🟢 [STEP #002] | Dist: 1.00m | Facing: NORTH | Press: 1013.25 hPa

🟢 [STEP #003] | Dist: 1.00m | Facing: EAST | Press: 1013.24 hPa

🟢 [STEP #004] | Dist: 1.00m | Facing: EAST | Press: 1013.25 hPa

🟢 [STEP #005] | Dist: 0.50m | Facing: EAST | Press: 1013.15 hPa

🟢 [STEP #006] | Dist: 0.50m | Facing: EAST | Press: 1013.00 hPa

🟢 [STEP #007] | Dist: 0.50m | Facing: SOUTH | Press: 1012.90 hPa

🟢 [STEP #008] | Dist: 1.00m | Facing: SOUTH | Press: 1012.89 hPa

🟢 [STEP #009] | Dist: 1.00m | Facing: SOUTH | Press: 1012.90 hPa

🟢 [STEP #010] | Dist: 1.00m | Facing: WEST | Press: 1012.88 hPa

🟢 [STEP #011] | Dist: 1.00m | Facing: WEST | Press: 1012.89 hPa

🟢 [STEP #012] | Dist: 0.50m | Facing: WEST | Press: 1012.70 hPa

🟢 [STEP #013] | Dist: 0.50m | Facing: NORTH | Press: 1012.55 hPa

🟢 [STEP #014] | Dist: 1.00m | Facing: NORTH | Press: 1012.53 hPa

🟢 [STEP #015] | Dist: 1.00m | Facing: NORTH | Press: 1012.54 hPa

Step 2: Python Script for Jupyter Notebook (plot_3d_from_file.py)

Run this complete code inside your Jupyter Notebook:

Python

import re

import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

from mpl_toolkits.mplot3d import Axes3D

# ==============================================================================

# 1. PARSE LOG FILE AUTOMATICALLY

# ==============================================================================

LOG_FILE_PATH = "sensor_output.txt"

def parse_sensor_log(file_path):

parsed_records = []

# Direction conversion mapping

cardinal_map = {

"N": 0.0, "NORTH": 0.0,

"NE": 45.0, "NORTH-EAST": 45.0,

"E": 90.0, "EAST": 90.0,

"SE": 135.0, "SOUTH-EAST": 135.0,

"S": 180.0, "SOUTH": 180.0,

"SW": 225.0, "SOUTH-WEST": 225.0,

"W": 270.0, "WEST": 270.0,

"NW": 315.0, "NORTH-WEST": 315.0

}

with open(file_path, "r") as f:

lines = f.readlines()

for idx, line in enumerate(lines):

line = line.strip()

if not line:

continue

# Regex extraction patterns (Flexible matching for different text formats)

dist_match = re.search(r'(?:Dist|distance|dis)[\s:]*([0-9.]+)\s*m?', line, re.IGNORECASE)

dir_match = re.search(r'(?:Facing|Direction|direction|dire)[\s:]*([A-Za-z0-9-]+)', line, re.IGNORECASE)

press_match = re.search(r'(?:Press|Pressure|pressure)[\s:]*([0-9.]+)\s*(?:hPa|Pa)?', line, re.IGNORECASE)

if dist_match and dir_match and press_match:

dist = float(dist_match.group(1))

dir_raw = dir_match.group(1).upper()

press = float(press_match.group(1))

# Convert Pa to hPa if pressure is logged in Pascals (e.g. 101325 Pa -> 1013.25 hPa)

if press > 2000.0:

press /= 100.0

# Convert direction string to degree angle

if dir_raw in cardinal_map:

heading_deg = cardinal_map[dir_raw]

else:

try:

heading_deg = float(dir_raw)

except ValueError:

heading_deg = 0.0

parsed_records.append({

"step": len(parsed_records) + 1,

"distance": dist,

"direction_str": dir_raw,

"heading_deg": heading_deg,

"pressure_hpa": press

})

return pd.DataFrame(parsed_records)

# Extract data into Pandas DataFrame

df = parse_sensor_log(LOG_FILE_PATH)

if df.empty:

print(f"[!] Error: Could not extract valid step lines from '{LOG_FILE_PATH}'. Check file format.")

else:

print(f" Successfully extracted {len(df)} steps from '{LOG_FILE_PATH}'.\n")

# ==============================================================================

# 2. CARTESIAN POSITION COMPUTATION (X, Y, Z)

# ==============================================================================

# Lock Ground Baseline Pressure P0 from the first line

P0 = df["pressure_hpa"].iloc[0]

# Calculate Relative Height Z (Meters) and Floor Index

# Hypsometric Formula: Z = 44330 * (1 - (P / P0)^0.1903)

df["Z_meters"] = 44330.0 * (1.0 - np.power((df["pressure_hpa"] / P0), 0.1903))

df["floor_index"] = np.round(df["Z_meters"] / 3.0).astype(int)

# Compute X and Y Coordinates using Trigonometric Dead-Reckoning

x_coords = [0.0]

y_coords = [0.0]

for i in range(len(df)):

dist = df.loc[i, "distance"]

rad = np.radians(df.loc[i, "heading_deg"])

new_x = x_coords[-1] + dist * np.sin(rad)

new_y = y_coords[-1] + dist * np.cos(rad)

x_coords.append(new_x)

y_coords.append(new_y)

df["X_pos"] = x_coords[1:]

df["Y_pos"] = y_coords[1:]

# Print calculated navigation summary

print("=== EXTRACTED 3D NAVIGATION LOG ===")

print(df[["step", "direction_str", "distance", "pressure_hpa", "X_pos", "Y_pos", "Z_meters", "floor_index"]])

# ==============================================================================

# 3. 3D VISUALIZATION GRAPH

# ==============================================================================

fig = plt.figure(figsize=(12, 9))

ax = fig.add_subplot(111, projection='3d')

# Include starting origin point (0, 0, Z0)

X_plot = np.insert(df["X_pos"].values, 0, 0.0)

Y_plot = np.insert(df["Y_pos"].values, 0, 0.0)

Z_plot = np.insert(df["Z_meters"].values, 0, df["Z_meters"].iloc[0])

# Plot 3D trajectory

sc = ax.scatter(X_plot, Y_plot, Z_plot, c=Z_plot, cmap='coolwarm', s=60, edgecolors='k', zorder=5)

ax.plot(X_plot, Y_plot, Z_plot, color='black', linestyle='--', linewidth=2, alpha=0.7, label='Walk Trajectory')

# Highlight Start (Green) and End (Red)

ax.scatter(X_plot[0], Y_plot[0], Z_plot[0], color='green', s=150, marker='^', label='Start (Origin)')

ax.scatter(X_plot[-1], Y_plot[-1], Z_plot[-1], color='red', s=150, marker='v', label='Current Position')

# Draw Floor Planes for Visual Clarity (3 meters per floor)

min_x, max_x = min(X_plot) - 1, max(X_plot) + 1

min_y, max_y = min(Y_plot) - 1, max(Y_plot) + 1

xx, yy = np.meshgrid(np.linspace(min_x, max_x, 10), np.linspace(min_y, max_y, 10))

unique_floors = df["floor_index"].unique()

colors = ['gray', 'cyan', 'orange', 'purple']

for floor in unique_floors:

floor_z = floor * 3.0

color = colors[floor % len(colors)]

ax.plot_surface(xx, yy, np.full_like(xx, floor_z), alpha=0.15, color=color)

ax.text(min_x, max_y, floor_z + 0.2, f"  FLOOR {floor} ({floor_z:.1f}m)", color='darkblue', fontweight='bold')

# Annotate Step Numbers along path

for i, txt in enumerate(range(len(X_plot))):

if txt == 0: continue

ax.text(X_plot[i], Y_plot[i], Z_plot[i] + 0.1, f" S{txt}", fontsize=8)

# Format Axes and Titles

ax.set_title("3D Indoor Trajectory Extracted From File", fontsize=14, fontweight='bold')

ax.set_xlabel("X Position (Meters)", fontweight='bold')

ax.set_ylabel("Y Position (Meters)", fontweight='bold')

ax.set_zlabel("Height Z (Meters)", fontweight='bold')

cbar = fig.colorbar(sc, ax=ax, shrink=0.5, aspect=10)

cbar.set_label('Elevation (Meters)', fontweight='bold')

ax.legend(loc='upper left')

plt.tight_layout()

plt.show()

How to use this:

- Paste your terminal logs into sensor_output.txt.

- Run the script in Jupyter Notebook.

- The script automatically uses Regex to find all step numbers, distances, directions, and pressure readings, builds the 3D grid, and renders the multi-floor trajectory plot!