Code explanation
This program is a complete dead-reckoning pedometer and indoor navigation system written in MicroPython.

It connects a microcontroller (like a Raspberry Pi Pico) to two hardware sensors via an I2C communication bus:

- ICP10111: A high-precision barometric pressure sensor (tracks air pressure/altitude).

- LSM303DLHC: A combined 3-axis accelerometer (detects steps and movement) and a 3-axis magnetometer (acts as an electronic compass to determine heading).

Here is the code broken down section-by-section, line-by-line, so you can understand exactly how it works.

Part 1: Imports & Hardware Pin Definitions

Python

from machine import Pin, I2C

import utime

import math

- from machine import Pin, I2C: Imports hardware control features from MicroPython. Pin handles microcontroller pins, while I2C manages the two-wire communication protocol used by the sensors.

- import utime: Imports time functions (delays, measuring elapsed milliseconds).

- import math: Imports mathematical functions like sine, cosine, square roots, and angle conversions.

Python

# Hardware Configuration

I2C_ID = 0

SDA_PIN = 0

SCL_PIN = 1

I2C_FREQ = 100000

- I2C_ID = 0: Tells the system to use Hardware I2C Controller #0 on the board.

- SDA_PIN = 0: Assigns GPIO Pin 0 as the SDA (Data) line.

- SCL_PIN = 1: Assigns GPIO Pin 1 as the SCL (Clock) line.

- I2C_FREQ = 100000: Sets the data transfer speed to 100 kHz (Standard I2C speed).

Python

# LSM303 Addresses

ACC_ADDRESS = 0x19

MAG_ADDRESS = 0x1E

# BARO Address

BARO_ADDRESS = 0x63

- These are hexadecimal hardware addresses (I2C addresses) that let the microcontroller target specific sensors:

  - 0x19: LSM303 Accelerometer.

  - 0x1E: LSM303 Magnetometer (Compass).

  - 0x63: ICP10111 Barometer.

Part 2: ICP10111 Barometer Sensor Driver Class

The ICP10111 sensor requires factory-calibration parameters read from its One-Time Programmable (OTP) memory to calculate accurate atmospheric pressure.

Python

class ICP10111:

def __init__(self, i2c_bus):

self.i2c = i2c_bus

self.c1 = self.c2 = self.c3 = self.c4 = 0

self.calibrated = False

self.p_Pa_calib = [45000.0, 80000.0, 105000.0]

self.LUT_lower = 3.5 * (1 << 20)

self.LUT_upper = 11.5 * (1 << 20)

self.quadr_factor = 1.0 / 16777216.0

self.offst_factor = 2048.0

- __init__(self, i2c_bus): Sets up default values when creating a barometer object:

  - Stores the I2C connection handle.

  - Sets calibration variables c1 to c4 to 0.

  - Sets calibrated = False.

  - p_Pa_calib: Reference pressure levels in Pascals (45 kPa, 80 kPa, 105 kPa).

  - Sets lookup table boundaries (LUT_lower, LUT_upper) and math scaling factors used by the manufacturer's conversion formula.

Python

def init_sensor(self):

try:

self.i2c.writeto(BARO_ADDRESS, bytes([0x80, 0x5D]))

utime.sleep_ms(10)

self.i2c.writeto(BARO_ADDRESS, bytes([0xC5, 0x95, 0x00, 0x66, 0x9C]))

utime.sleep_ms(5)

otp = []

for _ in range(4):

self.i2c.writeto(BARO_ADDRESS, bytes([0xC7, 0xF7]))

data = self.i2c.readfrom(BARO_ADDRESS, 3)

if not data or len(data) < 3: return False

val = (data[0] << 8) | data[1]

if val >= 32768: val -= 65536

otp.append(val)

self.c1, self.c2, self.c3, self.c4 = otp

self.calibrated = True

print(f"[ICP-10111] OTP Loaded -> C1={self.c1}, C2={self.c2}, C3={self.c3}, C4={self.c4}")

return True

except: return False

- init_sensor(): Communicates with the barometer to read factory calibration coefficients:

  - Sends commands 0x80, 0x5D and 0xC5, 0x95... to wake up OTP memory reading.

  - Loops 4 times to read 4 calibration constants (c1, c2, c3, c4).

  - Combines high and low bytes: (data[0] << 8) | data[1].

  - Converts unsigned numbers to signed integers: if val >= 32768: val -= 65536.

  - Saves calibration coefficients and prints them to the terminal.

Python

def_calc_const(self, p_Pa, LUT):

denom_c = (p_LUT[2]*(p_Pa[0]-p_Pa[1]) + p_LUT[0]*(p_Pa[1]-p_Pa[2]) + p_LUT[1]*(p_Pa[2]-p_Pa[0]))

if abs(denom_c) < 1e-6: return None, None, None

C = (p_LUT[0]*p_LUT[1]*(p_Pa[0]-p_Pa[1]) + p_LUT[1]*p_LUT[2]*(p_Pa[1]-p_Pa[2]) + p_LUT[2]*p_LUT[0]*(p_Pa[2]-p_Pa[0])) / denom_c

denom_a = (p_LUT[0] - p_LUT[1])

if abs(denom_a) < 1e-6: return None, None, None

A = (p_Pa[0]*p_LUT[0] - p_Pa[1]*p_LUT[1] - (p_Pa[1]-p_Pa[0])*C) / denom_a

B = (p_Pa[0]-A)*(p_LUT[0]+C)

return A, B, C

- def_calc_const(): A internal mathematical solver helper that calculates polynomial transfer coefficients (A, B, C) based on sensor curve lookup values (LUT) and standard pressure parameters.

Python

def get_pressure(self):

if not self.calibrated and not self.init_sensor(): return None

try:

self.i2c.writeto(BARO_ADDRESS, bytes([0x68, 0x25]))

utime.sleep_ms(15)

res = self.i2c.readfrom(BARO_ADDRESS, 9)

if not res or len(res) < 9: return None

T_LSB = (res[0] << 8) | res[1]

p_LSB = (res[3] << 16) | (res[4] << 8) | res[6]

temperature = -45.0 + (175.0 * T_LSB / 65535.0)

t = float(T_LSB - 32768)

s1 = self.LUT_lower + float(self.c1 * t - (t**2) * self.quadr_factor)

s2 = self.offst_factor * self.c4 + float(self.c2 * t - (t**2) * self.quadr_factor)

s3 = self.LUT_upper + float(self.c3 * t - (t**2) * self.quadr_factor)

A, B, C = self._calc_const(self.p_Pa_calib, [s1, s2, s3])

if A is None: return None

return A + (B / (C + float(p_LSB)))

except: return None

- get_pressure(): Requests raw temperature and pressure readings from the sensor:

  - Command 0x68, 0x25 tells the sensor to trigger a measurement in low-noise mode.

  - Reads 9 raw bytes.

  - Converts bit readings into temperature (​∘C) and raw pressure counts (p_LSB).

  - Applies factory calibration math (s1,s2,s3) to output corrected pressure in Pascals (Pa).

Part 3: LSM303 Register Definitions & Configuration

Python

# LSM303 REGISTERS & SETTINGS

CTRL_REG1_A = 0x20

CTRL_REG4_A = 0x23

OUT_X_L_A = 0x28

AUTO_INCREMENT = 0x80

CRA_REG_M = 0x00

CRB_REG_M = 0x01

MR_REG_M = 0x02

OUT_X_H_M = 0x03

- Configuration registers inside the LSM303 chip:

  - CTRL_REG1_A, CTRL_REG4_A: Set sample speed and sensitivity for the accelerometer.

  - OUT_X_L_A: Output start address for raw accelerometer axis measurements.

  - CRA_REG_M, CRB_REG_M, MR_REG_M: Setup mode and gain for the compass (magnetometer).

Python

# TUNING PARAMETERS

STRIDE_LENGTH_M = 0.5

STEP_COOLDOWN_MS = 350

ACCEL_THRESHOLD = 150

ALPHA = 0.3

- System tuning variables:

  - STRIDE_LENGTH_M = 0.5: Assumes each detected step covers 0.5 meters (50 cm).

  - STEP_COOLDOWN_MS = 350: Prevents double-counting step triggers within 350 ms.

  - ACCEL_THRESHOLD = 150: Sensitivity threshold for acceleration magnitude to trigger a step.

  - ALPHA = 0.3: Smoothing factor for an Exponential Moving Average filter (reduces sensor noise).

Python

# Global Variables

x_bias, y_bias, z_bias = 0, 0, 0

mag_x_bias, mag_y_bias = 0, 0

total_steps = 0

total_distance = 0.0

last_step_time = 0

prev_activity = "UNKNOWN"

pos_x, pos_y = 0.0, 0.0

last_heading = 0.0

accel_buffer = []

buffer_size = 15

baseline_pressure = 0.0

i2c = I2C(I2C_ID, freq=I2C_FREQ)

- Global tracking state:

  - x_bias, y_bias, z_bias, mag_x_bias, mag_y_bias: Calibration offset constants.

  - total_steps, total_distance: Cumulative step count and distance traveled in meters.

  - pos_x, pos_y: Current X,Y coordinate estimate on a 2D plane (Dead Reckoning).

  - i2c = I2C(...): Initializes the I2C bus object on hardware.

Part 4: Helper Functions & Sensor Readers

Python

def write_reg(addr, reg, value):

try:

i2c.writeto_mem(addr, reg, bytes([value]))

return True

except OSError: return False

def read_regs(addr, reg, length):

try:

return i2c.readfrom_mem(addr, reg, length)

except OSError: return None

- write_reg / read_regs: Helper functions to read/write hardware registers over I2C.

Python

def signed_12bit(val):

val >>= 4

if val >= 2048: val -= 4096

return val

def signed_16bit(val):

if val >= 32768: val -= 65536

return val

- signed_12bit / signed_16bit: Conversion functions that transform raw binary data into positive/negative signed numbers.

Python

def init_lsm303dlhc():

write_reg(ACC_ADDRESS, CTRL_REG1_A, 0x57) # 100Hz output, all axes enabled

write_reg(ACC_ADDRESS, CTRL_REG4_A, 0x08) # High resolution mode

write_reg(MAG_ADDRESS, CRA_REG_M, 0x14)    # 30Hz compass data update rate

write_reg(MAG_ADDRESS, CRB_REG_M, 0x20)    # Gain setting

write_reg(MAG_ADDRESS, MR_REG_M, 0x00)     # Continuous conversion mode

- init_lsm303dlhc(): Powers up accelerometer and magnetometer chips, configuring them to stream live continuous data.

Python

def read_accel():

data = read_regs(ACC_ADDRESS, OUT_X_L_A | AUTO_INCREMENT, 6)

if data is None: return None

return (

signed_12bit((data[1] << 8) | data[0]),

signed_12bit((data[3] << 8) | data[2]),

signed_12bit((data[5] << 8) | data[4])

)

def read_mag():

data = read_regs(MAG_ADDRESS, OUT_X_H_M, 6)

if data is None: return None

return (

signed_16bit((data[0] << 8) | data[1]),

signed_16bit((data[4] << 8) | data[5]),

signed_16bit((data[2] << 8) | data[3])

)

- read_accel() & read_mag(): Reads raw 6-byte data streams (X,Y,Z axes) from both sensors and converts them into numeric standard integers.

Part 5: Calibration & Heading Functions

Python

def calibrate_sensors(baro):

global x_bias, y_bias, z_bias, mag_x_bias, mag_y_bias, baseline_pressure

print("\n" + "="*60)

print("STEP 1/2: Keep board completely FLAT & STILL on table...")

print("="*60)

sx = sy = sz = p_sum = count = 0

for _ in range(40):

acc = read_accel()

p = baro.get_pressure()

if acc and p:

sx += acc[0]; sy += acc[1]; sz += acc[2]

p_sum += p; count += 1

utime.sleep_ms(30)

if count > 0:

x_bias = sx / count

y_bias = sy / count

z_bias = (sz / count) - 1024  # Removes 1G gravity constant offset

baseline_pressure = p_sum / count

print(f"-> Baseline Pressure Locked: {baseline_pressure:.2f} Pa")

- calibrate_sensors() - Step 1:

  - Asks the user to hold the board flat and motionless.

  - Takes 40 samples of accelerometer and barometer values to calculate system zero offsets (x_bias, y_bias, z_bias) and stores starting atmospheric pressure (baseline_pressure).

Python

print("STEP 2/2: PICK UP BOARD & ROTATE IN 3D 'FIGURE-8' FOR 7 SECONDS!")

min_x = min_y = 32767

max_x = max_y = -32768

start_time = utime.ticks_ms()

while utime.ticks_diff(utime.ticks_ms(), start_time) < 7000:

m = read_mag()

if m:

if m[0] < min_x: min_x = m[0]

if m[0] > max_x: max_x = m[0]

if m[1] < min_y: min_y = m[1]

if m[1] > max_y: max_y = m[1]

utime.sleep_ms(20)

mag_x_bias = (max_x + min_x) / 2.0

mag_y_bias = (max_y + min_y) / 2.0

print("-> Magnetometer Biases Calibrated Successfully!\n")

- calibrate_sensors() - Step 2:

  - Asks user to rotate the device in a figure-8 motion for 7 seconds.

  - Captures maximum and minimum compass magnetic field values along X and Y axes to neutralize local magnetic interference (hard-iron distortion calibration).

Python

def get_heading(mx, my):

mx_c = mx - mag_x_bias

my_c = my - mag_y_bias

h = math.degrees(math.atan2(my_c, mx_c))

return h if h >= 0 else h + 360

def get_cardinal(h):

if (h >= 337.5) or (h < 22.5): return "N"

if 22.5 <= h < 67.5: return "NE"

if 67.5 <= h < 112.5: return "E"

if 112.5 <= h < 157.5: return "SE"

if 157.5 <= h < 202.5: return "S"

if 202.5 <= h < 247.5: return "SW"

if 247.5 <= h < 292.5: return "W"

return "NW"

- get_heading(): Subtracts magnetic offset bias and converts 2D compass vectors into standard degree angles (0∘ to 360∘) using atan2.

- get_cardinal(): Maps degree angles to traditional compass cardinal directions (N, NE, E, SE, S, SW, W, NW).

Part 6: Main Execution Loop (Motion, Dead Reckoning & Output)

Python

def main():

global total_steps, total_distance, last_step_time, pos_x, pos_y, last_heading, prev_activity

init_lsm303dlhc()

baro = ICP10111(i2c)

if not baro.init_sensor():

print("Warning: Barometer Initialization Failed!")

calibrate_sensors(baro)

smooth_accel = 0

print("\n--- System Online ---")

- Initialization inside main(): Starts hardware sensors, performs sensor calibration steps, and initializes movement smoothing filters.

Python

while True:

try:

acc_vals = read_accel()

mag_vals = read_mag()

if not acc_vals or not mag_vals: continue

# Accel Processing

ax = acc_vals[0] - x_bias

ay = acc_vals[1] - y_bias

az = acc_vals[2] - z_bias

raw_mag = math.sqrt(ax**2 + ay**2 + az**2)

- while True:: Runs an infinite operational monitoring loop:

  - Reads current accelerometer and magnetometer data.

  - Subtracts calibration bias offsets from raw values.

  - Computes total vector acceleration magnitude: raw_mag=ax2+ay2+az2.

Python

# Smooth Filter

smooth_accel = (ALPHA * raw_mag) + ((1 - ALPHA) * smooth_accel)

accel_buffer.append(smooth_accel)

if len(accel_buffer) > buffer_size: accel_buffer.pop(0)

current_time = utime.ticks_ms()

avg_buffer = sum(accel_buffer) / len(accel_buffer)

- Applies Exponential Moving Average (EMA) smoothing to eliminate high-frequency noise/jitter, storing values inside a sliding 15-sample window buffer.

Python

# Step Detection

if smooth_accel > ACCEL_THRESHOLD:

if utime.ticks_diff(current_time, last_step_time) > STEP_COOLDOWN_MS:

if smooth_accel > (avg_buffer * 1.4):

last_step_time = current_time

total_steps += 1

total_distance += STRIDE_LENGTH_M

curr_h = get_heading(mag_vals[0], mag_vals[1])

rad = math.radians(curr_h)

pos_x += STRIDE_LENGTH_M * math.sin(rad)

pos_y += STRIDE_LENGTH_M * math.cos(rad)

last_heading = curr_h

- Step Trigger Logic:

  - Checks if current acceleration spike exceeds threshold (ACCEL_THRESHOLD).

  - Ensures enough time elapsed since last step (STEP_COOLDOWN_MS).

  - Ensures acceleration is 1.4× greater than the rolling average (confirming an active footstep).

  - Dead Reckoning Computation: Reads current compass heading angle, converts it to radians, and updates position coordinates (posx, posy) using trigonometry:

ΔX=stride×sinθ, ΔY=stride×cosθ

Python

# Barometer Reading

pressure_pa = baro.get_pressure()

pressure_hpa = (pressure_pa / 100.0) if pressure_pa else None

p_display = f"{round(pressure_hpa, 2)} hPa" if pressure_hpa else "Err"

# Print Status

print(f"STEP #{total_steps} | {get_cardinal(curr_h)} | Dist: {round(total_distance, 1)}m | Pos: ({round(pos_x, 1)}, {round(pos_y, 1)})")

print(f"     [ENV] Pressure: {p_display}")

- Formats environmental pressure (Pascals converted to hPa) and prints steps, compass direction, total distance, coordinates, and ambient air pressure to screen.

Python

# Activity State Classifier

activity = "SITTING" if smooth_accel < 30 else "STILL" if smooth_accel < 80 else "WALKING"

if activity != prev_activity:

print(f"\n>>> ACTIVITY: {activity} <<<\n")

prev_activity = activity

utime.sleep_ms(20)

except KeyboardInterrupt:

break

except Exception as e:

print("Error:", e)

if __name__ == "__main__":

main()

- Activity State Classifier: Classifies user physical state into "SITTING", "STILL", or "WALKING" based on baseline acceleration force level, printing a notification whenever activity state changes.

- if __name__ == "__main__": main(): Entry point that executes the main() function when the script runs.

same explain function by function
below


Here is the complete line-by-line breakdown for the entire codebase across all five images.

This program is a **dead-reckoning pedometer and indoor positioning system** written in MicroPython. It uses an **LSM303 accelerometer/magnetometer** (to track steps and compass direction) and an **ICP10111 barometric pressure sensor** (to monitor atmospheric pressure).

---

## Hardware Configuration & RT1064 Pin Setup

```python

from machine import Pin, I2C

import utime

import math

```

* **`from machine import Pin, I2C`**: Imports hardware peripheral control modules from MicroPython.

* **`import utime`**: Imports time delay and timestamping functions.

* **`import math`**: Imports mathematical operations (square roots, trigonometric functions, angle conversions).

```python

# Hardware Configuration

I2C_ID = 0

SDA_PIN = 0

SCL_PIN = 1

I2C_FREQ = 100000

```

* **`I2C_ID = 0`**: Selects I2C hardware bus index 0 on the board.

* **`SDA_PIN = 0`**: Assigns GPIO Pin 0 as the SDA (Serial Data) line.

* **`SCL_PIN = 1`**: Assigns GPIO Pin 1 as the SCL (Serial Clock) line.

* **`I2C_FREQ = 100000`**: Configures I2C bus speed to 100 kHz (Standard Mode).

> 💡 **NXP i.MX RT1064 EVK Note:** On your RT1064 board, check which **LPI2C** peripheral header you are plugged into (e.g., Arduino header J22/J23 or J24). Make sure `I2C_ID` (e.g. `0` or `1`) and `SDA_PIN` / `SCL_PIN` match the board's pin layout.

```python

# LSM303 Addresses

ACC_ADDRESS = 0x19

MAG_ADDRESS = 0x1E

# BARO Address

BARO_ADDRESS = 0x63

```

* **`ACC_ADDRESS = 0x19`**: I2C bus device address for the LSM303 accelerometer.

* **`MAG_ADDRESS = 0x1E`**: I2C bus device address for the LSM303 magnetometer (compass).

* **`BARO_ADDRESS = 0x63`**: I2C bus device address for the ICP10111 barometer.

---

## 1. Class: `ICP10111` (Barometric Pressure Sensor Driver)

### Function 1: `__init__(self, i2c_bus)`

```python

class ICP10111:

def __init__(self, i2c_bus):

self.i2c = i2c_bus

self.c1 = self.c2 = self.c3 = self.c4 = 0

self.calibrated = False

self.p_Pa_calib = [45000.0, 80000.0, 105000.0]

self.LUT_lower = 3.5 * (1 << 20)

self.LUT_upper = 11.5 * (1 << 20)

self.quadr_factor = 1.0 / 16777216.0

self.offst_factor = 2048.0

```

* **`def __init__(self, i2c_bus):`**: Constructor method initialized when instantiating the barometer object.

* **`self.i2c = i2c_bus`**: Stores the active I2C bus handle internally.

* **`self.c1 = self.c2 = self.c3 = self.c4 = 0`**: Initializes 4 factory OTP calibration constants to zero.

* **`self.calibrated = False`**: Sets calibration flag status to `False`.

* **`self.p_Pa_calib = [45000.0, 80000.0, 105000.0]`**: Defines target standard reference pressures in Pascals ($45\text{ kPa}$, $80\text{ kPa}$, $105\text{ kPa}$).

* **`self.LUT_lower = 3.5 * (1 << 20)`**: Calculates lower curve scaling threshold ($3.5 \times 2^{20} = 3,670,016$).

* **`self.LUT_upper = 11.5 * (1 << 20)`**: Calculates upper curve scaling threshold ($11.5 \times 2^{20} = 12,058,624$).

* **`self.quadr_factor = 1.0 / 16777216.0`**: Defines quadratic math conversion constant ($\frac{1}{2^{24}}$).

* **`self.offst_factor = 2048.0`**: Sets offset scaling factor ($2^{11}$).

---

### Function 2: `init_sensor(self)`

```python

def init_sensor(self):

try:

self.i2c.writeto(BARO_ADDRESS, bytes([0x80, 0x5D]))

utime.sleep_ms(10)

self.i2c.writeto(BARO_ADDRESS, bytes([0xC5, 0x95, 0x00, 0x66, 0x9C]))

utime.sleep_ms(5)

otp = []

for _ in range(4):

self.i2c.writeto(BARO_ADDRESS, bytes([0xC7, 0xF7]))

data = self.i2c.readfrom(BARO_ADDRESS, 3)

if not data or len(data) < 3: return False

val = (data[0] << 8) | data[1]

if val >= 32768: val -= 65536

otp.append(val)

self.c1, self.c2, self.c3, self.c4 = otp

self.calibrated = True

print(f"[ICP-10111] OTP Loaded -> C1={self.c1}, C2={self.c2}, C3={self.c3}, C4={self.c4}")

return True

except: return False

```

* **`def init_sensor(self):`**: Reads OTP (One-Time Programmable) factory calibration parameters from the sensor.

* **`try:`**: Wraps commands in exception handling to catch I2C errors.

* **`self.i2c.writeto(BARO_ADDRESS, bytes([0x80, 0x5D]))`**: Sends activation command `0x80, 0x5D` to wake sensor OTP memory.

* **`utime.sleep_ms(10)`**: Delays 10 ms for sensor internal power-up.

* **`self.i2c.writeto(BARO_ADDRESS, bytes([0xC5, 0x95, 0x00, 0x66, 0x9C]))`**: Sends OTP read initialization sequence.

* **`utime.sleep_ms(5)`**: Waits 5 ms for register lock.

* **`otp = []`**: Creates an empty list to collect calibration coefficients.

* **`for _ in range(4):`**: Runs a 4-iteration loop to pull the 4 calibration parameters.

* **`self.i2c.writeto(BARO_ADDRESS, bytes([0xC7, 0xF7]))`**: Requests next OTP register byte.

* **`data = self.i2c.readfrom(BARO_ADDRESS, 3)`**: Reads 3 bytes (2 data bytes + 1 CRC byte).

* **`if not data or len(data) < 3: return False`**: Validates reception; returns `False` if read failed.

* **`val = (data[0] << 8) | data[1]`**: Combines high and low bytes into a single 16-bit word.

* **`if val >= 32768: val -= 65536`**: Converts unsigned 16-bit integer to a signed integer (Two's complement).

* **`otp.append(val)`**: Appends signed integer value to `otp` array.

* **`self.c1, self.c2, self.c3, self.c4 = otp`**: Unpacks calibration list into individual variables `c1, c2, c3, c4`.

* **`self.calibrated = True`**: Sets internal flag to signal successful calibration load.

* **`print(...)`**: Displays retrieved calibration coefficients in the console terminal.

* **`return True`**: Signals initialization success.

* **`except: return False`**: Handles bus failure gracefully by returning `False`.

---

### Function 3: `_calc_const(self, p_Pa, p_LUT)`

```python

def _calc_const(self, p_Pa, p_LUT):

denom_c = (p_LUT[2]*(p_Pa[0]-p_Pa[1]) + p_LUT[0]*(p_Pa[1]-p_Pa[2]) + p_LUT[1]*(p_Pa[2]-p_Pa[0]))

if abs(denom_c) < 1e-6: return None, None, None

C = (p_LUT[0]*p_LUT[1]*(p_Pa[0]-p_Pa[1]) + p_LUT[1]*p_LUT[2]*(p_Pa[1]-p_Pa[2]) + p_LUT[2]*p_LUT[0]*(p_Pa[2]-p_Pa[0])) / denom_c

denom_a = (p_LUT[0] - p_LUT[1])

if abs(denom_a) < 1e-6: return None, None, None

A = (p_Pa[0]*p_LUT[0] - p_Pa[1]*p_LUT[1] - (p_Pa[1]-p_Pa[0])*C) / denom_a

B = (p_Pa[0]-A)*(p_LUT[0]+C)

return A, B, C

```

* **`def _calc_const(self, p_Pa, p_LUT):`**: Helper solver calculating polynomial conversion factors $A, B, C$.

* **`denom_c = ...`**: Solves mathematical matrix denominator term for constant $C$.

* **`if abs(denom_c) < 1e-6: return None, None, None`**: Prevents division by zero errors if denominator approaches 0.

* **`C = ...`**: Computes polynomial coefficient $C$.

* **`denom_a = (p_LUT[0] - p_LUT[1])`**: Computes denominator term for constant $A$.

* **`if abs(denom_a) < 1e-6: return None, None, None`**: Prevents division by zero error.

* **`A = ...`**: Computes polynomial coefficient $A$.

* **`B = (p_Pa[0]-A)*(p_LUT[0]+C)`**: Computes polynomial coefficient $B$.

* **`return A, B, C`**: Returns calculated tuple constants $(A, B, C)$.

---

### Function 4: `get_pressure(self)`

```python

def get_pressure(self):

if not self.calibrated and not self.init_sensor(): return None

try:

self.i2c.writeto(BARO_ADDRESS, bytes([0x68, 0x25]))

utime.sleep_ms(15)

res = self.i2c.readfrom(BARO_ADDRESS, 9)

if not res or len(res) < 9: return None

T_LSB = (res[0] << 8) | res[1]

p_LSB = (res[3] << 16) | (res[4] << 8) | res[6]

temperature = -45.0 + (175.0 * T_LSB / 65535.0)

t = float(T_LSB - 32768)

s1 = self.LUT_lower + float(self.c1 * t - (t**2) * self.quadr_factor)

s2 = self.offst_factor * self.c4 + float(self.c2 * t - (t**2) * self.quadr_factor)

s3 = self.LUT_upper + float(self.c3 * t - (t**2) * self.quadr_factor)

A, B, C = self._calc_const(self.p_Pa_calib, [s1, s2, s3])

if A is None: return None

return A + (B / (C + float(p_LSB)))

except: return None

```

* **`def get_pressure(self):`**: Reads sensor hardware registers and computes calibrated atmospheric pressure in Pascals.

* **`if not self.calibrated and not self.init_sensor(): return None`**: Ensures calibration sequence is complete before reading.

* **`try:`**: Wraps execution in exception handler.

* **`self.i2c.writeto(BARO_ADDRESS, bytes([0x68, 0x25]))`**: Sends command `0x68, 0x25` to trigger a low-noise conversion measurement cycle.

* **`utime.sleep_ms(15)`**: Waits 15 ms for hardware sample completion.

* **`res = self.i2c.readfrom(BARO_ADDRESS, 9)`**: Reads 9 raw data bytes from sensor.

* **`if not res or len(res) < 9: return None`**: Returns `None` if incomplete packet received.

* **`T_LSB = (res[0] << 8) | res[1]`**: Reconstructs 16-bit raw temperature value.

* **`p_LSB = (res[3] << 16) | (res[4] << 8) | res[6]`**: Reconstructs 24-bit raw pressure value.

* **`temperature = -45.0 + (175.0 * T_LSB / 65535.0)`**: Converts raw reading to temperature in $^\circ\text{C}$.

* **`t = float(T_LSB - 32768)`**: Computes temperature differential constant $t$.

* **`s1 = ...`**: Computes lower temperature scaling curve $s_1$.

* **`s2 = ...`**: Computes offset scaling curve $s_2$.

* **`s3 = ...`**: Computes upper temperature scaling curve $s_3$.

* **`A, B, C = self._calc_const(...)`**: Calculates dynamic equation parameters $A, B, C$.

* **`if A is None: return None`**: Returns `None` if solver fails.

* **`return A + (B / (C + float(p_LSB)))`**: Computes final calibrated atmospheric pressure in Pascals ($\text{Pa}$) via equation $P = A + \frac{B}{C + P_{\text{LSB}}}$.

* **`except: return None`**: Catches read errors.

---

## 2. LSM303 Register Settings & Global Setup

```python

# LSM303 REGISTERS & SETTINGS

CTRL_REG1_A = 0x20

CTRL_REG4_A = 0x23

OUT_X_L_A = 0x28

AUTO_INCREMENT = 0x80

CRA_REG_M = 0x00

CRB_REG_M = 0x01

MR_REG_M = 0x02

OUT_X_H_M = 0x03

```

* **`CTRL_REG1_A = 0x20`**: Accelerometer Control Register 1 (Data rate and axis enable).

* **`CTRL_REG4_A = 0x23`**: Accelerometer Control Register 4 (Full scale selection & HR mode).

* **`OUT_X_L_A = 0x28`**: Starting register address for accelerometer multi-byte X/Y/Z reads.

* **`AUTO_INCREMENT = 0x80`**: High bit command to automatically auto-increment register address during multi-byte reads.

* **`CRA_REG_M`, `CRB_REG_M`, `MR_REG_M**`: Magnetometer control registers for sample rate, gain, and mode.

* **`OUT_X_H_M = 0x03`**: Starting register address for magnetometer data reads.

```python

# TUNING PARAMETERS

STRIDE_LENGTH_M = 0.5

STEP_COOLDOWN_MS = 350

ACCEL_THRESHOLD = 150

ALPHA = 0.3

```

* **`STRIDE_LENGTH_M = 0.5`**: Assumed distance covered per step ($0.5\text{ meters}$).

* **`STEP_COOLDOWN_MS = 350`**: Minimum allowed time ($350\text{ ms}$) between steps to reject false triggers.

* **`ACCEL_THRESHOLD = 150`**: Acceleration magnitude threshold required to trigger a step.

* **`ALPHA = 0.3`**: Smoothing constant factor for Exponential Moving Average noise filtering.

```python

# Global Variables

x_bias, y_bias, z_bias = 0, 0, 0

mag_x_bias, mag_y_bias = 0, 0

total_steps = 0

total_distance = 0.0

last_step_time = 0

prev_activity = "UNKNOWN"

pos_x, pos_y = 0.0, 0.0

last_heading = 0.0

accel_buffer = []

buffer_size = 15

baseline_pressure = 0.0

i2c = I2C(I2C_ID, freq=I2C_FREQ)

```

* Sets up global variables tracking sensor offsets (`x_bias`...`mag_y_bias`), total step count, calculated coordinates (`pos_x`, `pos_y`), and instantiates the main system `i2c` bus object.

---

## 3. Helper Functions & LSM303 Low-Level Drivers

### Function 5: `write_reg(addr, reg, value)`

```python

def write_reg(addr, reg, value):

try:

i2c.writeto_mem(addr, reg, bytes([value]))

return True

except OSError: return False

```

* **`def write_reg(addr, reg, value):`**: Writes a single byte `value` to register address `reg` on I2C device `addr`.

* **`try:`**: Start error checking block.

* **`i2c.writeto_mem(addr, reg, bytes([value]))`**: MicroPython system call writing data byte to slave register.

* **`return True`**: Returns `True` on successful write.

* **`except OSError: return False`**: Returns `False` on bus transmission error.

---

### Function 6: `read_regs(addr, reg, length)`

```python

def read_regs(addr, reg, length):

try:

return i2c.readfrom_mem(addr, reg, length)

except OSError: return None

```

* **`def read_regs(addr, reg, length):`**: Reads `length` byte sequence from register address `reg` on device `addr`.

* **`try:`**: Start error checking block.

* **`return i2c.readfrom_mem(...)`**: Reads byte array from memory register and returns it.

* **`except OSError: return None`**: Returns `None` if read fails.

---

### Function 7: `signed_12bit(val)`

```python

def signed_12bit(val):

val >>= 4

if val >= 2048: val -= 4096

return val

```

* **`def signed_12bit(val):`**: Converts left-aligned 12-bit sensor output to a signed integer.

* **`val >>= 4`**: Bit-shifts 4 bits right to strip off unused trailing zeros.

* **`if val >= 2048: val -= 4096`**: Converts unsigned range ($2048 \dots 4095$) into negative signed range (Two's complement).

* **`return val`**: Returns processed signed integer.

---

### Function 8: `signed_16bit(val)`

```python

def signed_16bit(val):

if val >= 32768: val -= 65536

return val

```

* **`def signed_16bit(val):`**: Converts unsigned 16-bit word ($0 \dots 65535$) into signed integer ($-32768 \dots +32767$).

* **`if val >= 32768: val -= 65536`**: Subtracts $2^{16}$ if number represents negative value.

* **`return val`**: Returns signed 16-bit integer.

---

### Function 9: `init_lsm303dlhc()`

```python

def init_lsm303dlhc():

write_reg(ACC_ADDRESS, CTRL_REG1_A, 0x57)

write_reg(ACC_ADDRESS, CTRL_REG4_A, 0x08)

write_reg(MAG_ADDRESS, CRA_REG_M, 0x14)

write_reg(MAG_ADDRESS, CRB_REG_M, 0x20)

write_reg(MAG_ADDRESS, MR_REG_M, 0x00)

```

* **`def init_lsm303dlhc():`**: Initializes control registers for accelerometer and magnetometer hardware.

* **`write_reg(ACC_ADDRESS, CTRL_REG1_A, 0x57)`**: Powers on accelerometer at $100\text{ Hz}$ data rate with X, Y, Z axes enabled.

* **`write_reg(ACC_ADDRESS, CTRL_REG4_A, 0x08)`**: Enables High-Resolution mode and $\pm 2g$ scale range.

* **`write_reg(MAG_ADDRESS, CRA_REG_M, 0x14)`**: Sets magnetometer update rate to $30\text{ Hz}$.

* **`write_reg(MAG_ADDRESS, CRB_REG_M, 0x20)`**: Configures magnetic sensor gain ($\pm 1.3\text{ Gauss}$).

* **`write_reg(MAG_ADDRESS, MR_REG_M, 0x00)`**: Configures magnetometer for continuous measurement mode.

---

### Function 10: `read_accel()`

```python

def read_accel():

data = read_regs(ACC_ADDRESS, OUT_X_L_A | AUTO_INCREMENT, 6)

if data is None: return None

return (

signed_12bit((data[1] << 8) | data[0]),

signed_12bit((data[3] << 8) | data[2]),

signed_12bit((data[5] << 8) | data[4])

)

```

* **`def read_accel():`**: Reads current 3-axis acceleration measurements.

* **`data = read_regs(...)`**: Reads 6 registers starting from `OUT_X_L_A` with auto-increment flag.

* **`if data is None: return None`**: Aborts if I2C read fails.

* **`return (...)`**: Returns tuple containing signed 12-bit acceleration values for $(X, Y, Z)$ axes.

---

### Function 11: `read_mag()`

```python

def read_mag():

data = read_regs(MAG_ADDRESS, OUT_X_H_M, 6)

if data is None: return None

return (

signed_16bit((data[0] << 8) | data[1]),

signed_16bit((data[4] << 8) | data[5]),

signed_16bit((data[2] << 8) | data[3])

)

```

* **`def read_mag():`**: Reads current 3-axis magnetic measurements.

* **`data = read_regs(...)`**: Reads 6 registers starting from address `OUT_X_H_M`.

* **`if data is None: return None`**: Aborts if I2C read fails.

* **`return (...)`**: Combines high and low bytes and returns signed 16-bit values for $(X, Y, Z)$ axes (Note: LSM303 hardware output order places Z before Y).

---

## 4. Calibration & Heading Processing

### Function 12: `calibrate_sensors(baro)`

```python

def calibrate_sensors(baro):

global x_bias, y_bias, z_bias, mag_x_bias, mag_y_bias, baseline_pressure

print("\n" + "="*60)

print("STEP 1/2: Keep board completely FLAT & STILL on table...")

print("="*60)

sx = sy = sz = p_sum = count = 0

for _ in range(40):

acc = read_accel()

p = baro.get_pressure()

if acc and p:

sx += acc[0]; sy += acc[1]; sz += acc[2]

p_sum += p; count += 1

utime.sleep_ms(30)

if count > 0:

x_bias = sx / count

y_bias = sy / count

z_bias = (sz / count) - 1024

baseline_pressure = p_sum / count

print(f"-> Baseline Pressure Locked: {baseline_pressure:.2f} Pa")

print("STEP 2/2: PICK UP BOARD & ROTATE IN 3D 'FIGURE-8' FOR 7 SECONDS!")

min_x = min_y = 32767

max_x = max_y = -32768

start_time = utime.ticks_ms()

while utime.ticks_diff(utime.ticks_ms(), start_time) < 7000:

m = read_mag()

if m:

if m[0] < min_x: min_x = m[0]

if m[0] > max_x: max_x = m[0]

if m[1] < min_y: min_y = m[1]

if m[1] > max_y: max_y = m[1]

utime.sleep_ms(20)

mag_x_bias = (max_x + min_x) / 2.0

mag_y_bias = (max_y + min_y) / 2.0

print("-> Magnetometer Biases Calibrated Successfully!\n")

```

* **`def calibrate_sensors(baro):`**: Performs 2-step calibration for static gravity/pressure baseline and dynamic magnetic hard-iron interference.

* **`global ...`**: Unpacks global calibration variables into scope.

* **`sx = sy = sz = p_sum = count = 0`**: Resets reading accumulators.

* **`for _ in range(40):`**: Collects 40 stationary baseline samples.

* **`if acc and p:`**: Ensures sensor readings are valid before accumulating.

* **`sx += acc[0]...`**: Adds readings to cumulative sums.

* **`x_bias = sx / count`**: Computes static mean acceleration bias along X-axis.

* **`y_bias = sy / count`**: Computes static mean acceleration bias along Y-axis.

* **`z_bias = (sz / count) - 1024`**: Computes Z-axis bias, subtracting nominal $1\text{G}$ gravity force constant ($1024$ LSB in $\pm 2g$ mode).

* **`baseline_pressure = p_sum / count`**: Computes locked baseline atmospheric pressure.

* **`while utime.ticks_diff(...) < 7000:`**: Runs a 7-second active loop asking user to rotate device in a figure-8 motion.

* **`if m[0] < min_x: min_x = m[0] ...`**: Tracks maximum and minimum raw magnetic readings along X and Y axes.

* **`mag_x_bias = (max_x + min_x) / 2.0`**: Calculates magnetic offset center for X-axis.

* **`mag_y_bias = (max_y + min_y) / 2.0`**: Calculates magnetic offset center for Y-axis.

---

### Function 13: `get_heading(mx, my)`

```python

def get_heading(mx, my):

mx_c = mx - mag_x_bias

my_c = my - mag_y_bias

h = math.degrees(math.atan2(my_c, mx_c))

return h if h >= 0 else h + 360

```

* **`def get_heading(mx, my):`**: Calculates compass heading angle in degrees from raw magnetometer inputs.

* **`mx_c = mx - mag_x_bias`**: Subtracts X-axis hard-iron calibration offset.

* **`my_c = my - mag_y_bias`**: Subtracts Y-axis hard-iron calibration offset.

* **`h = math.degrees(math.atan2(my_c, mx_c))`**: Computes 2D vector angle using arc tangent `atan2` and converts radians to degrees ($-180^\circ \dots +180^\circ$).

* **`return h if h >= 0 else h + 360`**: Normalizes output angle into $0^\circ \dots 360^\circ$ circle range.

---

### Function 14: `get_cardinal(h)`

```python

def get_cardinal(h):

if (h >= 337.5) or (h < 22.5): return "N"

if 22.5 <= h < 67.5: return "NE"

if 67.5 <= h < 112.5: return "E"

if 112.5 <= h < 157.5: return "SE"

if 157.5 <= h < 202.5: return "S"

if 202.5 <= h < 247.5: return "SW"

if 247.5 <= h < 292.5: return "W"

return "NW"

```

* **`def get_cardinal(h):`**: Converts numerical compass angle `h` into standard cardinal directions (**N**, **NE**, **E**, **SE**, **S**, **SW**, **W**, **NW**).

---

## 5. Main Loop & Dead Reckoning Execution

### Function 15: `main()`

```python

def main():

global total_steps, total_distance, last_step_time, pos_x, pos_y, last_heading, prev_activity

init_lsm303dlhc()

baro = ICP10111(i2c)

if not baro.init_sensor():

print("Warning: Barometer Initialization Failed!")

calibrate_sensors(baro)

smooth_accel = 0

print("\n--- System Online ---")

```

* **`def main():`**: Main application setup and operational program execution logic.

* Initializes LSM303, constructs barometer driver, runs calibration routine, and sets initial variables.

```python

while True:

try:

acc_vals = read_accel()

mag_vals = read_mag()

if not acc_vals or not mag_vals: continue

# Accel Processing

ax = acc_vals[0] - x_bias

ay = acc_vals[1] - y_bias

az = acc_vals[2] - z_bias

raw_mag = math.sqrt(ax**2 + ay**2 + az**2)

```

* **`while True:`**: Begins infinite operational polling loop.

* Reads raw sensor data and subtracts zero-motion calibration biases.

* **`raw_mag = math.sqrt(...)`**: Calculates 3D acceleration vector magnitude $\sqrt{ax^2 + ay^2 + az^2}$.

```python

# Smooth Filter

smooth_accel = (ALPHA * raw_mag) + ((1 - ALPHA) * smooth_accel)

accel_buffer.append(smooth_accel)

if len(accel_buffer) > buffer_size: accel_buffer.pop(0)

current_time = utime.ticks_ms()

avg_buffer = sum(accel_buffer) / len(accel_buffer)

```

* Applies Exponential Moving Average (EMA) smoothing filter to clean raw acceleration signal.

* Maintains a rolling buffer array of the 15 most recent acceleration samples.

```python

# Step Detection

if smooth_accel > ACCEL_THRESHOLD:

if utime.ticks_diff(current_time, last_step_time) > STEP_COOLDOWN_MS:

if smooth_accel > (avg_buffer * 1.4):

last_step_time = current_time

total_steps += 1

total_distance += STRIDE_LENGTH_M

curr_h = get_heading(mag_vals[0], mag_vals[1])

rad = math.radians(curr_h)

pos_x += STRIDE_LENGTH_M * math.sin(rad)

pos_y += STRIDE_LENGTH_M * math.cos(rad)

last_heading = curr_h

```

* **Step Detection & Dead Reckoning**:

1. Checks if acceleration magnitude spike exceeds `ACCEL_THRESHOLD`.

2. Ensures enough time ($350\text{ ms}$) has elapsed since last detected step.

3. Checks if acceleration spike is $1.4\times$ larger than local average (verifying true footstep impact).

4. Increments step count (`total_steps`) and total distance (`total_distance`).

5. Computes $(X, Y)$ coordinate displacement using dead reckoning trigonometry:

$$\Delta X = \text{stride} \times \sin(\theta), \quad \Delta Y = \text{stride} \times \cos(\theta)$$

```python

# Barometer Reading

pressure_pa = baro.get_pressure()

pressure_hpa = (pressure_pa / 100.0) if pressure_pa else None

p_display = f"{round(pressure_hpa, 2)} hPa" if pressure_hpa else "Err"

# Print Status

print(f"STEP #{total_steps} | {get_cardinal(curr_h)} | Dist: {round(total_distance, 1)}m | Pos: ({round(pos_x, 1)}, {round(pos_y, 1)})")

print(f"     [ENV] Pressure: {p_display}")

```

* Samples pressure in Pascals, converts to hectopascals ($\text{hPa}$), and prints current positioning metrics and environmental pressure data to console.

```python

# Activity State

activity = "SITTING" if smooth_accel < 30 else "STILL" if smooth_accel < 80 else "WALKING"

if activity != prev_activity:

print(f"\n>>> ACTIVITY: {activity} <<<\n")

prev_activity = activity

utime.sleep_ms(20)

except KeyboardInterrupt:

break

except Exception as e:

print("Error:", e)

```

* Classifies current user motion state into `"SITTING"`, `"STILL"`, or `"WALKING"`.

* Prints status message when activity mode transitions.

* Delays $20\text{ ms}$ per cycle ($50\text{ Hz}$ update rate) and catches keyboard interrupts for clean exit.

```python

if __name__ == "__main__":

main()

```

* Checks if script is executed directly and triggers `main()`.