1st code for graph

import re

)

else:

text = (

f"<b>Step #{df.loc[i, 'step']}</b><br>"

f"<b>Pressure:</b> {df.loc[i, 'pressure']:.2f} hPa<br>"

f"<b>Smoothed Elevation (Z):</b> {df.loc[i, 'Z_meters']:.2f} m<br>"

f"<b>Pos (X, Y):</b> ({df.loc[i, 'X_pos']:.2f}m, {df.loc[i, 'Y_pos']:.2f}m)<br>"

f"<b>Heading:</b> {df.loc[i, 'direction']} ({df.loc[i, 'heading_deg']:.0f}°)<br>"

f"<b>Total Dist:</b> {df.loc[i, 'distance']:.1f} m"

)

hover_texts.append(text)

# ==============================================================================

# 7. INTERACTIVE 3D GRAPH GENERATION

# ==============================================================================

fig = go.Figure()

# 3D Trajectory Path Line

fig.add_trace(go.Scatter3d(

x=X_plot, y=Y_plot, z=Z_plot,

mode='lines',

line=dict(color='black', width=4),

name='Smoothed Path',

hoverinfo='skip'

))

# 3D Colored Step Nodes

fig.add_trace(go.Scatter3d(

x=X_plot, y=Y_plot, z=Z_plot,

mode='markers',

marker=dict(

size=7,

color=Z_plot,

colorscale='Viridis',

colorbar=dict(title="Elevation Z (m)"),

showscale=True,

line=dict(color='black', width=1)

),

text=hover_texts,

hoverinfo='text',

name='Steps'

))

# Start Point Marker (Green Diamond)

fig.add_trace(go.Scatter3d(

x=[X_plot[0]], y=[Y_plot[0]], z=[Z_plot[0]],

mode='markers',

marker=dict(size=12, color='green', symbol='diamond'),

text=[hover_texts[0]],

hoverinfo='text',

name='Start'

))

# Final Destination End Marker (Red Square)

fig.add_trace(go.Scatter3d(

x=[X_plot[-1]], y=[Y_plot[-1]], z=[Z_plot[-1]],

mode='markers',

marker=dict(size=12, color='red', symbol='square'),

text=[hover_texts[-1]],

hoverinfo='text',

name='Destination'

))

# 3D Scene Configuration

fig.update_layout(

title=f"<b>3D Indoor Navigation Path</b><br><sup>Transition: {floor_transition} | Net ΔP: {net_dp:.2f} hPa</sup>",

scene=dict(

xaxis=dict(title='X Position (Meters)', backgroundcolor="rgb(245, 245, 245)"),

yaxis=dict(title='Y Position (Meters)', backgroundcolor="rgb(245, 245, 245)"),

zaxis=dict(title='Height / Elevation Z (Meters)', backgroundcolor="rgb(230, 240, 250)"),

aspectmode='data'

),

margin=dict(l=0, r=0, b=0, t=60),

legend=dict(x=0.01, y=0.99)

)

# Render inside Jupyter Notebook Cell Output

pyo.iplot(fig)

2nd code for graph

import re

# ==============================================================================

# 6. PRESSURE VS DISTANCE GRAPH

# ==============================================================================

fig_pressure = go.Figure()

fig_pressure.add_trace(

go.Scatter(

x=df["Distance"],

y=df["Pressure"],

mode="lines+markers",

name="Pressure",

text=[

f"Step {step}<br>"

f"Direction: {direction}<br>"

f"Distance: {distance:.1f} m<br>"

f"Pressure: {pressure:.2f} hPa"

for step, direction, distance, pressure

in zip(

df["Step"],

df["Direction"],

df["Distance"],

df["Pressure"]

)

],

hoverinfo="text"

)

)

fig_pressure.update_layout(

title="Pressure vs Walking Distance",

xaxis_title="Distance (m)",

yaxis_title="Pressure (hPa)",

template="plotly_white"

)

pyo.iplot(fig_pressure)

# ==============================================================================

# 7. 2D WALKING PATH

# ==============================================================================

fig_path = go.Figure()

fig_path.add_trace(

go.Scatter(

x=df["X"],

y=df["Y"],

mode="lines+markers+text",

text=df["Step"].astype(str),

textposition="top center",

name="Walking Path"

)

)

fig_path.update_layout(

title="2D Walking Path",

xaxis_title="X Position (m)",

yaxis_title="Y Position (m)",

template="plotly_white",

xaxis=dict(scaleanchor="y")

)

pyo.iplot(fig_path)

# ==============================================================================

# 8. PRESSURE CHANGE PER STEP

# ==============================================================================

fig_step_pressure = go.Figure()

fig_step_pressure.add_trace(

go.Bar(

x=df["Step"],

y=df["Pressure"],

text=df["Pressure"].round(2),

textposition="outside",

name="Pressure"

)

)

fig_step_pressure.update_layout(

title="Pressure at Each Step",

xaxis_title="Step Number",

yaxis_title="Pressure (hPa)",

template="plotly_white"

)

pyo.iplot(fig_step_pressure)

3rd graph code
import re

# 7. INTERACTIVE 3D GRAPH GENERATION

# ==============================================================================

fig = go.Figure()

# 3D Trajectory Path Line

fig.add_trace(go.Scatter3d(

x=X_plot, y=Y_plot, z=Z_plot,

mode='lines',

line=dict(color='black', width=4),

name='Smoothed Path',

hoverinfo='skip'

))

# 3D Colored Step Nodes

fig.add_trace(go.Scatter3d(

x=X_plot, y=Y_plot, z=Z_plot,

mode='markers',

marker=dict(

size=7,

color=Z_plot,

colorscale='Viridis',

colorbar=dict(title="Elevation Z (m)"),

showscale=True,

line=dict(color='black', width=1)

),

text=hover_texts,

hoverinfo='text',

name='Steps'

))

# Start Point Marker (Green Diamond)

fig.add_trace(go.Scatter3d(

x=[X_plot[0]], y=[Y_plot[0]], z=[Z_plot[0]],

mode='markers',

marker=dict(size=12, color='green', symbol='diamond'),

text=[hover_texts[0]],

hoverinfo='text',

name='Start'

))

# Final Destination End Marker (Red Square)

fig.add_trace(go.Scatter3d(

x=[X_plot[-1]], y=[Y_plot[-1]], z=[Z_plot[-1]],

mode='markers',

marker=dict(size=12, color='red', symbol='square'),

text=[hover_texts[-1]],

hoverinfo='text',

name='Destination'

))

# 3D Scene Configuration

fig.update_layout(

title=f"<b>3D Indoor Navigation Path</b><br><sup>Transition: {floor_transition} | Net ΔP: {net_dp:.2f} hPa</sup>",

scene=dict(

xaxis=dict(title='X Position (Meters)', backgroundcolor="rgb(245, 245, 245)"),

yaxis=dict(title='Y Position (Meters)', backgroundcolor="rgb(245, 245, 245)"),

zaxis=dict(title='Height / Elevation Z (Meters)', backgroundcolor="rgb(230, 240, 250)"),

aspectmode='data'

),

margin=dict(l=0, r=0, b=0, t=60),

legend=dict(x=0.01, y=0.99)

)

# Render inside Jupyter Notebook Cell Output

pyo.iplot(fig)

c pins
To route both USB Charging (Power/Data) and Analog Audio (Headphones) through a single USB Type-C port, the system relies on the USB Type-C standard's Audio Adapter Accessory Mode combined with an analog multiplexer/switch and a CC (Configuration Channel) detection controller.

1. How It Works (The Core Concept)

A standard USB-C port does not have dedicated headphone pins. Instead, it reuses existing high-speed pins when an analog audio adapter is plugged in:

| Pin Type | Standard Mode (Charging / Data) | Audio Accessory Mode (Headphones) |
| --- | --- | --- |
| DP (D+) | USB 2.0 Data Positive | Audio Left Channel (L) |
| DN (D-) | USB 2.0 Data Negative | Audio Right Channel (R) |
| SBU1 / SBU2 | Sideband Use (DisplayPort/Aux) | Microphone (MIC) / Ground (AGND) |
| CC1 / CC2 | Connection & Orientation Detect | Pulled down with Ra resistors (signals audio plug) |
| VBUS / GND | Power supply (5V - 20V) | Power input for charging (via Y-splitters) |

2. End-to-End Circuit Architecture

Here is the block-level flow of the circuit:

Plaintext

+------------------------------------------------------+

|                  USB Type-C Receptacle               |

+-----+---------+---------+---------+------------+-----+

|         |         |         |            |

VBUS      CC1/CC2   D+/D-    SBU1/SBU2     GND

|         |         |         |            |

v         v         |         |            v

+-------+--+   +--+----+    |         |        +---+----+

| Charger  |   | CC    |    |         |        | System |

| IC / PMIC|   | Logic |    |         |        | Ground |

+----------+   +----+--+    |         |        +--------+

|       |         |

Switch Control |         |

v       v         v

+---------------------------------------+

| USB / Audio Analog Switch (e.g. FSA4480)|

+-------------------+-------------------+

|

+----------------+----------------+

|                                 |

v                                 v

+----------------------+          +----------------------+

|   USB PHY / SoC      |          | Audio Codec / DAC    |

| (High-Speed Data D+/D-)|        | (Headphone Out L/R,  |

|                      |          |  MIC In, AGND Sense) |

+----------------------+          +----------------------+

3. Step-by-Step Circuit Design

Step 1: CC Line Detection (Controller)

- What it does: Senses what type of accessory is plugged in.

- Mechanism: When an analog audio adapter is inserted, both CC1 and CC2 pins are pulled down to GND via internal termination resistors (Ra≤800 Ω).

- Action: The CC controller detects this condition and sends an interrupt/I2C command to flip the analog switch from USB Mode to Audio Mode.

Step 2: Analog Audio Switch IC (e.g., FSA4480 or TS3USB3000)

- High-speed D+ and D- lines cannot be directly wired to both the SoC USB controller and the Audio DAC without causing signal corruption.

- You need a dedicated low-distortion, negative-swing analog multiplexer:

  - Input A (Normal): D+/D- routed to the processor's USB 2.0 PHY.

  - Input B (Audio): D+ routed to DAC Left Channel, D- routed to DAC Right Channel.

  - SBU Routing: SBU1 and SBU2 are switched to the Audio Codec’s MIC and AGND/GND lines (supporting OMTP and CTIA jack standards).

Step 3: Audio Codec & Negative Voltage Swing

- Standard audio waveforms swing above and below 0V (AC-coupled or ground-centered).

- The audio switch must support negative voltage swing (down to -1.5V or -2V) without clipping or blowing up on a single positive battery supply (3.3V-4.2V).

Step 4: Power Management & Protection (ESD / OVP)

- VBUS Line: Goes directly through an Over-Voltage Protection (OVP) IC into the PMIC / Charger IC.

- ESD Protection: USB-C lines are exposed to the outside world, so high-speed low-capacitance ESD diodes must be placed right near the receptacle pins on D+, D-, CC, and SBU lines.

4. Summary of Key Components Needed

- USB Type-C 24-pin or 16-pin Receptacle: Physical port.

- Audio/USB Multiplexer Switch (e.g., ON Semi FSA4480 / TI TS3USB221): Handles the physical routing between USB data and audio signals.

- Audio DAC / Codec (e.g., Realtek / Cirrus Logic / Qualcomm WCD): Generates analog audio and handles headset mic/button sensing.

- CC Controller / Type-C Port Controller (TCPC): Detects connection states and handles USB-PD negotiation if charging simultaneously.

- Battery Charger IC / PMIC: Manages power delivery safely over VBUS.

Here is a detailed, pin-by-pin schematic hookup guide to design a shared USB Data / Fast Charging / Analog Audio interface using the industry-standard ON Semiconductor FSA4480 (USB-C Analog Audio Switch with Integrated High-Voltage Protection) and a standard USB Type-C Receptacle.

1. System Architecture & IC Role

The FSA4480 sits directly between the USB-C receptacle and the internal subsystems (Audio Codec, USB PHY, and Baseband/Host). It handles:

- Automatic negative voltage swing for audio (VSW down to -3V).

- Switching between USB 2.0 Differential Data and Audio L/R lines.

- Cross-switching the MIC and Audio Ground (AGND) between SBU1 and SBU2 (supporting both OMTP and CTIA 3.5mm headsets automatically).

- High-voltage protection on audio lines (up to 20V short-to-VBUS tolerance).

2. Pin-by-Pin Wiring & Connection Matrix

A. USB Type-C Connector to FSA4480 & Power

| USB-C Receptacle Pin | Signal Name | Connects To | Notes |
| --- | --- | --- | --- |
| A4, A9, B4, B9 | VBUS | OVP IC / PMIC (Charger) | Connect via decoupling cap (10μF/25V). |
| A1, A12, B1, B12 | GND | System GND Plane | Low impedance path. |
| A6 / B6 | DP (D+) | FSA4480 Pin: DP_R | USB D+ / Right Audio shared common port. |
| A7 / B7 | DN (D-) | FSA4480 Pin: DN_L | USB D- / Left Audio shared common port. |
| A8 | SBU1 | FSA4480 Pin: SBU1 | Switched to MIC or AGND internally. |
| B8 | SBU2 | FSA4480 Pin: SBU2 | Switched to AGND or MIC internally. |
| A5 | CC1 | CC Controller / PMIC CC1 | Pull-down / Type-C logic detect. |
| B5 | CC2 | CC Controller / PMIC CC2 | Pull-down / Type-C logic detect. |
| Shield Shell | SHIELD | Chassis GND | Connect through a 1 MΩ resistor in parallel with a 4.7 nF/1kV capacitor to system GND. |

B. FSA4480 to Internal Subsystems (Audio Codec & SoC USB)

| FSA4480 Pin | Pin Name | Connects To (Internal Line) | Function |
| --- | --- | --- | --- |
| VBAT / VDD | Power Input | System Power (2.7V-5.5V) | Decouple with 0.1μF+1μF caps to GND. |
| GND | Ground | System Digital Ground | Single-point connection. |
| DP_R | I/O Port | USB-C D+ Pin | Shared common side. |
| DN_L | I/O Port | USB-C D- Pin | Shared common side. |
| SBU1 / SBU2 | I/O Port | USB-C SBU1 & SBU2 | Shared sideband side. |
| HPR | Audio Input | Audio Codec: HEADPHONE_OUT_R | Analog Audio Right Channel. |
| HPL | Audio Input | Audio Codec: HEADPHONE_OUT_L | Analog Audio Left Channel. |
| USB_DP | USB Data | SoC / Processor: USB2.0_D+ | High-Speed USB 2.0 Differential Pair. |
| USB_DN | USB Data | SoC / Processor: USB2.0_D- | High-Speed USB 2.0 Differential Pair. |
| MIC | Audio In/Out | Audio Codec: MIC_IN / MICBIAS | Headset microphone circuit. |
| AGND | Analog Ground | Audio Codec: GND_SENSE / AGND | Audio ground reference path. |
| SCL / SDA | Control Bus | Host SoC: I2C_SCL / I2C_SDA | Register configuration and control (pull-ups 2.2 kΩ to 1.8V). |
| INTB | Interrupt | Host SoC: GPIO_INT | Active-low alert for plug/unplug & button press. |

3. Circuit Schematic Diagram (ASCII Representation)

Plaintext

+-------------------------------------------------------------------------+

|                          USB TYPE-C CONNECTOR                           |

+-----+---------+---------+---------+------------+------------+-----+-----+

|         |         |         |            |            |     |

VBUS      GND       CC1       CC2          DP           DN    SBU1  SBU2

|         |         |         |            |            |     |     |

v         v         |         |            |            |     |     |

+-------+  +----+       v         v            |            |     |     |

| PMIC/ |  | GND|  +--------------------+      |            |     |     |

|Charger|  |Plane  | TCPC / CC Detector |      |            |     |     |

+-------+  +----+  +---------+----------+      |            |     |     |

| I2C Interrrupt  |            |     |     |

v                 v            v     v     v

+-------------------------------------------------------------+

|                    FSA4480 SWITCH IC                        |

|                                                             |

| [Common]   DP_R             DN_L         SBU1        SBU2   |

|              |               |             |           |    |

|       +------+------+ +------+------+      |           |    |

|       |             | |             |      +----+ +----+    |

|   (USB Mode)    (Audio Mode)        (Cross Switch Matrix)   |

|       |             | |             |           |     |     |

|    USB_DP         HPR USB_DN       HPL         MIC   AGND   |

+-------+------------+-----+----------+-----------+------+----+

|            |     |          |           |      |

+------------+            |     +----+     |           |      |

|                         |          |     |           |      |

v                         |          v     |           |      |

+-------------------+               |   +------------+---+       |      |

|  SoC (Processor)  |               |   |  SoC USB 2.0   |       |      |

|  USB High-Speed   |               |   |  Controller    |       |      |

+-------------------+               |   +----------------+       |      |

|                            |      |

v                            v      v

+--------------------------------------------------+

|                AUDIO CODEC / DAC                 |

|                                                  |

|  • HP_OUT_R  • HP_OUT_L  • MIC_BIAS  • AGND_SNS  |

+--------------------------------------------------+

4. Operational Flow & Detection Sequence

- Idle State (No Accessory):

  - The switch defaults to High-Impedance or standard USB 2.0 data path.

- Accessory Insertion:

  - When an analog audio adapter is plugged in, both CC1 and CC2 are pulled down to ground via Ra resistors (≤800 Ω).

  - The CC controller recognizes this state as Audio Adapter Accessory Mode.

- Switch Activation:

  - The CC controller or Host SoC writes via I2C to the FSA4480 to flip the mux:

    - DP_R connects to HPR

    - DN_L connects to HPL

- CTIA vs. OMTP Auto-Detection:

  - Headset jacks have two ground/mic standards: CTIA (Sleeve = GND, Ring2 = MIC) and OMTP (Sleeve = MIC, Ring2 = GND).

  - The FSA4480 senses resistance on SBU1 and SBU2 and cross-routes them to MIC and AGND automatically, preventing inverted microphone/ground issues.

- Concurrent Fast Charging (Splitter Cable):

  - If a Y-splitter is used (Charger + Headphone), VBUS supplies 5V-20V straight to the charger IC while the DP/DN lines continue routing analog audio via the FSA4480.

5. Critical PCB Layout Rules

- Differential Impedance for USB: Keep USB_DP and USB_DN routed as a 90 Ω differential pair with matched trace lengths (<0.15 mm skew).

- Audio Trace Isolation: Route HPR, HPL, and MIC away from high-speed clocks and the switching power inductor of the PMIC to avoid audible hum or buzzing.

- ESD Placement: Place high-speed bidirectional ESD diodes (capacitance <0.5 pF) right next to the USB-C receptacle pins before the signals enter the FSA4480.

- Grounding: Keep AGND tied back to the Audio Codec's clean analog ground return before joining the main digital ground plane.

Here is how your phone knows what is plugged in and plays music, explained in simple, step-by-step words.

1. How the Phone Detects: "Is It a Charger or Earphones?"

Inside the USB-C port, there are two special "sensor" pins called CC1 and CC2 (Configuration Channel pins). The phone constantly monitors the voltage on these pins:

+-----------------------------+

|   Your Mobile Phone         |

|   (Checking CC Pin Voltage) |

+--------------+--------------+

|

+-----------------------+-----------------------+

|                                               |

Case 1: Charger Plugged In                      Case 2: Audio Plugged In

• Only ONE CC pin changes voltage.              • BOTH CC1 and CC2 drop to ground

• Phone thinks: "This is a Power Source!"       • Phone thinks: "This is an Earphone!"

|                                               |

v                                               v

Sends power to Battery                          Flips internal switch to Audio

- When you plug in a Charger: Only one CC pin drops to a specific voltage (Rd resistance). The phone immediately says: "A power source is connected, start charging."

- When you plug in Analog Earphones (or a 3.5mm-to-Type-C dongle): Both CC1 and CC2 pins drop to near ground (Ra resistance). The phone immediately says: "Both pins are grounded—this is an audio accessory, not a charger!"

2. How the Audio Data Reaches Your Ears

Once the phone knows earphones are connected, the data flows like this:

[ MP3 / Spotify Audio File ]

│

▼  (Digital 1s and 0s)

[ Phone Processor (SoC) ]

│

▼

[ Audio DAC / Codec ] ──► (Converts digital 1s & 0s into smooth sound waves)

│

▼  (Analog Sound Waves)

[ Internal Switch Chip ] ──► (Directs sound to the USB-C port pins)

│

▼

[ USB-C Port (D+ / D- Pins) ]

│

▼  (Travels down the earphone wire)

[ Earphone Speaker Driver ] ──► (Vibrates the magnet & diaphragm)

│

▼

🎵 Sound in your ears!

3. What Happens with Digital Type-C Earphones?

There are two kinds of USB-C earphones you buy in the market:

- Passive / Analog Earphones (Simple Type):

  - They have no brain of their own.

  - Your phone’s internal DAC creates the sound and sends actual sound waves through the USB-C pins.

- Active / Digital Earphones (With built-in DAC chip):

  - These earphones behave like a tiny USB computer.

  - The phone doesn't send sound waves; it sends digital data (1s and 0s) over USB.

  - A tiny chip inside the earphone plug converts that data into sound right next to your ears.

Summary in One Line

The phone looks at the CC pins to see the resistance (1 pin changed = charger, 2 pins changed = earphones). Once earphones are confirmed, an internal electronic switch reroutes the data lines directly to the phone's sound card (DAC), which pumps audio waves down the cable to the speakers.

Here is the complete, end-to-end technical breakdown of how an audio signal from an external microphone (via an analog headset/mic plugged into the USB-C port or an internal MEMS microphone) is captured, conditioned, digitized, processed, and handed off to the mobile operating system and applications.

1. End-to-End Signal Chain Architecture

[Acoustic Pressure Wave]

│

▼

[Transducer / Capsule] (ECM / MEMS)

│  ~0.5 mV to 50 mV (Weak AC Voltage)

▼

[USB-C SBU Pin + Switch] (FSA4480 Mux / Crosspoint)

│

▼

[Bias & DC-Blocking Circuit] (MICBIAS: ~1.8V - 2.8V DC + Coupling Cap)

│

▼

[Analog Front-End (AFE)] (PGA / Differential Pre-Amplifier)

│

▼

[Delta-Sigma (ΔΣ) ADC] (Analog-to-Digital Converter: Oversampling, Noise Shaping)

│  PDM or Multi-bit Stream

▼

[Decimation & Digital Filter Engine] (FIR/IIR Filters, High-Pass DC Filter)

│  Linear PCM (e.g., 24-bit / 48 kHz or 96 kHz)

▼

[Digital Audio Interface (DAI)] (I2S / TDM / SoundWire / SLIMbus)

│

▼

[SoC Audio DSP / Audio Engine] (AEC, Beamforming, Noise Suppression, EQ)

│  DMA Transfer

▼

[Application Processor / OS Framework] (ALSA / TinyALSA ➔ Audio HAL ➔ AudioFlinger / CoreAudio ➔ App)

2. Step-by-Step Technical Breakdown

Step 1: Acoustic to Electrical Transduction (Transducer Level)

External wired headsets generally use an Electret Condenser Microphone (ECM) or an analog MEMS Microphone:

- Mechanism: Sound waves create acoustic pressure against a flexible diaphragm separated by an air gap from a rigid backplate, forming a variable capacitor:

C=ε⋅Ad

- Impedance Converter: Because the capsule has an extremely high output impedance, an internal JFET (Junction Field Effect Transistor) inside the capsule buffers the signal.

- Power Requirement: The JFET requires an external DC bias voltage (MICBIAS) applied through a pull-up resistor (Rbias≈2.2 kΩ). The tiny fluctuations in diaphragm capacitance modulate the current through the JFET, producing an AC voltage ripple (Vsig≈1 mV-50 mV).

Step 2: Physical Layer Routing (USB-C & Analog Switch)

When connected over USB-C:

- Physical Pins: The microphone line comes through either SBU1 or SBU2 (depending on cable plug orientation).

- Auto-Mux (e.g., FSA4480):

  - The switch automatically identifies whether the accessory uses CTIA (Sleeve = GND, Ring2 = MIC) or OMTP (Sleeve = MIC, Ring2 = GND).

  - It routes the microphone signal to the internal MIC_IN line and ties the return ground to clean AGND (Analog Ground), preventing ground loop noise and crosstalk from headphone outputs.

Step 3: Analog Front-End (AFE) Conditioning

Inside the Audio Codec (e.g., Qualcomm WCD938x, Cirrus Logic CS47L35, or Realtek ALC5686):

- DC Bias Injection:

  - A low-noise, high-PSRR (Power Supply Rejection Ratio) LDO inside the codec generates MICBIAS (1.8 V-2.8 V).

- AC Coupling (DC Blocking):

  - A series capacitor (Cin≈1μF-2.2μF) strips off the high DC bias voltage, passing only the pure alternating audio signal.

- Programmable Gain Amplifier (PGA):

  - Amplifies the microvolt/millivolt signal by 0 dB to +30 dB to match the dynamic range and full-scale input (VFS) of the Analog-to-Digital Converter.

Step 4: Analog-to-Digital Conversion (ADC)

Most modern mobile audio codecs use a Multi-bit Delta-Sigma (ΔΣ) ADC:

Plaintext

Analog Input ──► (+) ──► [ Integrator(s) ] ──► [ Quantizer ] ──► PDM Bitstream

▲                                   │

│                                   │

└────────── [ 1-bit DAC ] ──────────┘

- Oversampling: The signal is sampled at a multiple of the target sample rate (e.g., 64×fs or 128×fs, running at several MHz).

- Noise Shaping: The ΔΣ modulator pushes quantization noise out of the audible frequency band (20 Hz-20 kHz) into high, inaudible radio frequencies.

- Decimation & Digital Filtering:

  - A CIC (Cascaded Integrator-Comb) and FIR decimation filter downsamples the high-rate bitstream to standard audio rates (e.g., 48 kHz / 24-bit PCM or 96 kHz / 24-bit PCM).

  - A digital High-Pass Filter (HPF) at 10 Hz-20 Hz rejects DC offsets and wind/pop low-frequency thumps.

Step 5: Chip-to-Chip Bus Transmission (Hardware Level)

The digitized audio frames leave the Audio Codec and travel to the Application Processor / SoC via digital serial audio buses:

- SoundWire (MIPI SoundWire) or SLIMbus: High-speed, multi-channel standard for low-latency bidirectional digital audio and control over 2 wires.

- I2S (Inter-IC Sound) / TDM (Time Division Multiplexing):

  - BCLK (Bit Clock): Synchronizes individual bits (BCLK=2×Bit Depth×fs).

  - LRCLK / WCLK (Word Clock / Frame Sync): Marks the start of Left/Right or channel time slots (48 kHz).

  - SDIN / DIN (Serial Data In): Carries the serialized 2's complement PCM binary stream.

Step 6: Digital Signal Processing (Audio DSP Subsystem)

Inside the SoC's dedicated low-power Audio DSP (e.g., Hexagon DSP on Snapdragon, Cadence Tensilica, or Apple Audio Engine):

- Acoustic Echo Cancellation (AEC): Prevents speaker output from looping back into the microphone.

- Active Noise Suppression (ANS) & Beamforming: If multiple mics exist (internal ambient mic + headset mic), spatial filtering algorithms cancel background noise.

- Automatic Gain Control (AGC) & Dynamic Range Compression (DRC): Normalizes fluctuating voice volume to prevent digital clipping (0 dBFS).

Step 7: OS Audio Pipeline & Memory Transfer (Software Level)

[ Audio DSP / DMA Engine ]

│ Writes to Ring Buffer (Circular RAM Buffer)

▼

[ Kernel Driver: ALSA / TinyALSA ] (Generates Hardware Interrupts)

│

▼

[ Audio Hardware Abstraction Layer (Audio HAL) ]

│

▼

[ Audio Framework: Android AudioFlinger / iOS AudioServer ]

│ Mixes / Routes / Resamples

▼

[ App Audio API (AAudio, Oboe, AVAudioEngine, MediaRecorder) ]

│

▼

[ User-Space Application (WhatsApp, Voice Recorder, Call App) ]

- Direct Memory Access (DMA): The DSP/Audio Controller streams PCM samples straight into a circular buffer in system RAM without CPU intervention.

- Interrupt Service Routine (ISR): When a buffer period (e.g., every 5ms or 128 samples) is filled, an interrupt notifies the kernel driver (ALSA / TinyALSA).

- Audio HAL (Hardware Abstraction Layer): Translates hardware-specific ALSA parameters to standard OS interfaces.

- OS Audio Engine (AudioFlinger on Android): Reads from the HAL, performs sample rate conversion if needed, enforces permissions, and passes the buffer over IPC/Binder to the application's recording API (AAudio, Oboe, or MediaRecorder).

D+ (Data Plus) and D- (Data Minus) are the core pair of communication lines used in all USB 2.0 standards, including the USB Type-C connector.

They serve two distinct purposes depending on how you use your phone:

- In USB Data Mode: A high-speed digital communications channel using Differential Signaling.

- In USB-C Audio Mode: Physical conduits carrying Analog Stereo Sound to your headphones.

1. How D+ and D- Work in Digital Mode: Differential Signaling

Most simple circuits measure voltage on a single wire relative to Ground (Single-Ended Signaling). USB doesn't do this for high-speed data because long cables pick up electromagnetic noise (from motors, Wi-Fi, radio waves).

Instead, USB uses Differential Signaling:

- D+ carries the normal data signal.

- D- carries the exact opposite (inverted) signal.

Signal "1" (J-State)               Signal "0" (K-State)

D+  ───┐           ┌─── (+3.3V)    D+  ─────────────┐     ┌─── (0V)

│           │                                │     │

└───────────┘ (0V)                           └─────┘

D-  ─────────────┐     ┌─── (0V)   D-  ───┐           ┌─── (+3.3V)

│     │                  │           │

└─────┘                  └───────────┘

Why use two opposite wires?

- Noise Cancellation (Common-Mode Rejection):

  - If external electrical interference hits the cable, it adds noise (+Vnoise) equally to both wires.

  - The receiver measures the difference between the two lines:

Vmeasured=VD++Vnoise-VD-+Vnoise=VD+-VD-

  - The noise completely cancels out mathematically.

- Lower Voltages & Higher Speeds:

  - In High-Speed mode (480 Mbps), the voltage swing drops from 3.3V down to just ±400mV, enabling ultra-fast transitions with minimal power and electromagnetic radiation.

2. Speed and Role Detection via Pull-Up Resistors

Before data starts streaming, the host (computer/phone) uses the DC state of D+ and D- to identify what speed device is connected:

| Device Type | Pull-up Configuration | What the Phone Detects |
| --- | --- | --- |
| Low-Speed USB (1.5 Mbps) | 1.5 kΩ pull-up on D- | D- line pulled high to 3.3V |
| Full-Speed USB (12 Mbps) | 1.5 kΩ pull-up on D+ | D+ line pulled high to 3.3V |
| High-Speed USB (480 Mbps) | Starts at Full-Speed, then "chirps" | D+ / D- negotiate down to 400mV differential |

3. How D+ and D- Are Repurposed for Audio (Audio Accessory Mode)

When you connect a passive USB-C to 3.5mm headphone adapter, the phone's CC pins tell the system to disconnect the USB controller from D+ and D-.

An internal switch (multiplexer) connects the phone's Audio DAC directly to these pins:

┌─── (USB Mode) ────► SoC USB 2.0 PHY (Digital 1s & 0s)

USB-C D+ Pin ───[ Mux ]──┤

└─── (Audio Mode) ──► Audio Right (R) Channel (Analog Waveform)

┌─── (USB Mode) ────► SoC USB 2.0 PHY (Digital 1s & 0s)

USB-C D- Pin ───[ Mux ]──┤

└─── (Audio Mode) ──► Audio Left (L) Channel (Analog Waveform)

- D+ becomes the Right Audio Channel (R).

- D- becomes the Left Audio Channel (L).

- The analog AC sound wave travels straight down the copper lines to the earphone speakers.

4. D+ and D- in Battery Charging (BC 1.2 & Fast Charging)

D+ and D- are also used by chargers to negotiate charging speeds:

- Standard USB 2.0 Port: D+ and D- remain separate → Max current is 500mA.

- Dedicated Charging Port (BC 1.2 Standard): The charger shorts D+ and D- together inside the wall brick → The phone senses the short and pulls up to 1.5A-2.4A.

- Quick Charge (Qualcomm QC 2.0/3.0): The phone applies specific voltages to D+ and D- (e.g., D+=0.6V, D-=0.6V) to tell the charger to bump the main power line up from 5V to 9V, 12V, or 20V.

Designing a 5G smartphone requires managing multiple radios, gigabit-speed digital data lines, and extreme radio frequency (RF) bands ranging from legacy 2G/3G/4G to 5G Sub-6 GHz (FR1) and 5G mmWave (FR2: 24 GHz – 40+ GHz).

1. Overall System Architecture & Block Diagram

A modern 5G smartphone motherboard is split into four primary hardware domains:

Plaintext

+-------------------------------------------------------------+

|                  APPLICATION PROCESSOR (AP)                 |

|  (CPU Cores, GPU, NPU/AI, RAM/LPDDR5, UFS Storage, Display) |

+------------------------------+------------------------------+

| PCIe / Inter-Chip Interface

v

+-------------------------------------------------------------+

|                       5G MODEM (BASEBAND)                   |

|  (Channel Coding, Layer 1-3 Stack, PHY, FFT/IFFT, Beamforming)|

+------------------------------+------------------------------+

| I/Q Samples (Analog/DigRF)

v

+-------------------------------------------------------------+

|                      RF TRANSCEIVER (RFIC)                  |

|   • Up/Down Converters (Mixers, PLL/LO) • ADC/DAC • AGC     |

+---------------+-----------------------------+---------------+

|                             |

Sub-6 GHz RF Line |                             | IF / Control (mmWave)

v                             v

+---------------------------------------------+   +------------------------------------+

|       RF FRONT-END MODULE (RFFE / FEM)      |   |        5G mmWave MODULE (AiP)      |

|  • Power Amplifier (PA) + Envelope Tracker  |   |  • Up/Down Converter + Beamformer  |

|  • Low-Noise Amplifier (LNA)                |   |  • Integrated 4x4 / 8x8 Patch Array|

|  • SAW / BAW Filters & Duplexers            |   +-----------------+------------------+

|  • Antenna Tuning Switches (SPxT, Crossbar) |                     |

+----------------------+----------------------+                     |

| $50\ \Omega$ RF Traces                     | Integrated

v                                            v

[ Sub-6 Antennas ]                         [ mmWave Phased Array ]

2. The Core Hardware Subsystems

A. Baseband 5G Modem

- Role: Translates raw application data into modulated digital waveforms.

- Key Tasks: Handles 3GPP protocols, OFDMA modulation up to 4096-QAM, carrier aggregation (combining multiple frequency channels), and massive MIMO digital processing.

- Connection to Host AP: High-speed PCIe Gen 3/4 or unified memory bus (if integrated inside a single SoC like Snapdragon 8-series or MediaTek Dimensity).

B. RF Transceiver (RFIC)

- Role: Bridges the digital baseband domain to analog radio waves.

- In Transmission (TX): Converts digital I/Q (In-Phase / Quadrature) data streams to analog via high-speed DACs, mixes them with a Local Oscillator (LO) to the desired 5G carrier frequency (e.g., 3.5 GHz for band n78), and delivers the intermediate RF signal.

- In Reception (RX): Down-converts faint RF signals from the antenna down to baseband frequencies for the ADCs.

C. RF Front-End (RFFE) & Power Amplifiers (Sub-6 GHz)

The RFFE is the most component-dense area on the board:

- Power Amplifiers (PA): Typically built on GaAs (Gallium Arsenide) or GaN (Gallium Nitride) dies for high linearity and high power output (+23 dBm to +26 dBm).

- Envelope Tracking (ET) IC: Instead of supplying a constant 3.8 V to the PA, the ET PMIC dynamically varies the PA voltage rail to match the instantaneous amplitude peak of the 5G waveform, saving up to 30% battery power.

- Acoustic Wave Filters (SAW / BAW): Bulk Acoustic Wave (BAW) and Surface Acoustic Wave (SAW) filters isolate the target transmission/reception band while attenuating out-of-band noise and Wi-Fi/Bluetooth interference.

- Low Noise Amplifiers (LNAs): Boost ultra-weak incoming signals (<-100 dBm) with minimal added noise figure (NF<1.5 dB).

D. 5G mmWave Modules (Antenna-in-Package - AiP)

At frequencies like 28 GHz and 39 GHz, traditional copper PCB traces suffer severe signal loss (attenuation).

- The Solution: The transceiver front-end, beamforming ICs, and patch antenna arrays are packaged together into a tiny module called AiP (Antenna-in-Package).

- Smartphones place 2 to 3 AiP modules along the perimeter (top and sides) so the signal is not blocked by the user’s hand.

- Analog Beamforming: The module uses phase shifters to steer focused radio beams toward the nearest cell tower electronically in real-time.

3. How the Schematic Circuit Is Designed

Hardware engineers build the circuit hierarchically using schematic capture tools (Cadence Allegro, Altium Designer, or Siemens Xpedition):

1. Power Tree Schematic & Decoupling

5G modems and PAs demand massive transient current spikes (up to 5A-10A during peak transmission).

- Multi-rail PMIC: Generates core rails (0.75V, 0.85V, 1.2V, 1.8V, 3.3V, VBAT).

- Decoupling Network: Multiple parallel MLCC ceramic capacitors (100nF, 1μF, 10μF of 0201 or 01005 metric package sizes) are placed immediately beside each supply pin to minimize parasitic loop inductance:

Lloop=μ0⋅Aw

2. High-Speed Digital Bus Schematics

- MIPI RFFE Bus: A dedicated multi-drop 2-wire serial bus (Clock + Bidirectional Data at up to 52 MHz) controlling LNAs, PAs, and switches with sub-microsecond latency.

- PCIe Gen 3/4: Connects AP and external modem using 85 Ω or 100 Ω differential pairs, AC-coupled via series 100nF capacitors.

3. RF Matching Network & Control

Between each RFFE IC, filter, and antenna feed point, engineers include a Pi (π) or T matching network consisting of precision inductors and capacitors:

Plaintext

Series Inductor (L1)

RF Out ────────────[ L1 ]────────────► Antenna Feed

│                │

[ C1 ]           [ C2 ]   (Shunt Capacitors to Ground)

│                │

GND              GND

- Purpose: Matches the complex load impedance (ZL) to the standard 50 Ω characteristic impedance across broad frequency bands to prevent standing waves and signal reflection (S11<-10 dB).

- Dynamic Antenna Tuners: Switched capacitor ICs dynamically compensate for detuning when the user holds the phone in different orientations.

4. High-Density PCB Layout Considerations

A 5G smartphone logic board cannot use standard PCB manufacturing; it uses mSAP (Modified Semi-Additive Process) and Any-Layer HDI (High-Density Interconnect) with 10 to 14 layers:

| Layer Parameter | 5G PCB Requirement | Engineering Reason |
| --- | --- | --- |
| RF Trace Geometry | Grounded Coplanar Waveguides (GCPW) | Shield high-frequency 50 Ω lines from adjacent digital buses. |
| Via Architecture | Microvias (laser-drilled, copper filled) | Eliminates dangling via stubs that create resonant reflections at GHz frequencies. |
| Shielding Cans | Sputtered Compartment Shielding (EMI Cans) | Prevents PA power emissions from jamming the LNA and GNSS/GPS receivers. |
| Thermal Dissipation | Copper coin inserts & graphite/vapor chambers | 5G processing generates concentrated heat (>45∘C); heat is piped away from the modem to the chassis. |

To activate the circuit for Audio Mode or Charging Mode, the system follows a hardware state machine driven by voltage/resistance detection on the CC pins, followed by I2C firmware commands to configure the analog switch (like the FSA4480) and power delivery ICs.

1. Activation Flow Overview

[ Accessory Plugged into Type-C ]

│

▼

[ CC Pin Voltage Sensing by TCPC ]

│

┌─────────────────────────┴─────────────────────────┐

│                                                   │

(Audio Accessory Detected)                          (Charger Detected)

Both CC1 & CC2 pulled to GND ($R_a$)               One CC pin pulled to $R_d$

│                                                   │

▼                                                   ▼

[ TCPC sends Interrupt (INTB) to SoC ]               [ USB-PD Negotiation begins ]

│                                                   │

▼                                                   ▼

[ SoC / Driver issues I2C Writes ]                  [ PMIC / Charger IC turns on ]

• Configure FSA4480 Switch                          • Enables VBUS Power Path

• Enable Audio Codec & Amp                          • Starts Fast Charge (5V–20V)

│                                                   │

▼                                                   ▼

[ 🎵 Sound Routed to D+/D- Pins ]                    [ 🔋 Battery Charges Safely ]

2. How to Activate Audio Accessory Mode

Step 1: Hardware Detection (CC Lines)

- The Type-C Port Controller (TCPC) applies an internal pull-up current source (Ip) or pull-up resistor (Rp) to both CC1 and CC2.

- When an analog audio adapter is plugged in, it contains two internal pull-down resistors (Ra≤800 Ω) connected to Ground:

  - Voltage on CC1 drops below 0.2 V

  - Voltage on CC2 drops below 0.2 V

- The TCPC detects VCC1<0.2 V AND VCC2<0.2 V simultaneously and asserts an active-low interrupt line (INTB) to wake up the Host SoC.

Step 2: Switch Configuration via I2C (e.g., FSA4480)

The Host SoC audio driver responds to the interrupt by sending I2C commands to the switch IC registers:

C

// Pseudo-code for Audio Switch Activation (FSA4480 Example)

// 1. Reset / Enable the Switch IC

i2c_write(FSA4480_ADDR, REG_RESET, 0x01);

// 2. Set Switch Path to Audio Mode (Disconnect USB D+/D-, connect HPL/HPR to DP/DN)

// Bit 0: Connect DN_L to HPL

// Bit 1: Connect DP_R to HPR

i2c_write(FSA4480_ADDR, REG_SWITCH_CTRL, 0x03);

// 3. Enable Audio MIC & Ground Routing

// Automatically senses whether plug is CTIA or OMTP standard

i2c_write(FSA4480_ADDR, REG_AUTO_DETECT, 0x01);

// 4. Unmute Audio Switch Output

i2c_write(FSA4480_ADDR, REG_MUTE_CTRL, 0x00);

Step 3: Turn On the Audio Subsystem

- Power up the Audio Codec / DAC and Headphone Output Stage (HPL_OUT, HPR_OUT).

- Power up MICBIAS (usually set to 2.2 V DC) to power the headset microphone capsule.

- Start the I2S / SoundWire digital audio clock and begin streaming PCM audio data.

3. How to Activate Charging Mode

Step 1: Hardware Sink/Source Detection

- When a charger/adapter is plugged in, the charger provides a pull-up (Rp) on one CC line, while the phone provides a pull-down resistor (Rd=5.1 kΩ) on both CC lines.

- The phone detects voltage on only one CC line (which also determines plug orientation: CC1 = normal, CC2 = flipped).

Step 2: USB Power Delivery (USB-PD) Protocol Handshake

- BMC Communication: The phone and charger communicate using Bi-phase Mark Code (BMC) data packets sent directly over the active CC line.

- Capabilities Exchange:

  - Charger sends: Source_Capabilities (e.g., 5V/3A, 9V/3A, 15V/3A, 20V/3.25A).

  - Phone requests: Request Message (e.g., asks for 9V/3A).

  - Charger acknowledges with Accept followed by PS_RDY (Power Supply Ready).

Step 3: Enabling the Power Path

- The OVP (Over-Voltage Protection) FET gate driver opens to let current flow from VBUS into the phone.

- The PMIC / Buck-Boost Charger IC (e.g., BQ25890 or similar) enables its switching converter and begins stepping down VBUS voltage to safely charge the lithium-ion battery (3.8V-4.45V).

4. Activating Both Simultaneously (Splitter / Dongle Mode)

When using a 2-in-1 USB-C splitter (Headphones + Charger connected at once):

- Power Path (VBUS): The charger's VBUS passes directly to the phone's charging IC for power.

- Data Lines (DP/DN): The analog switch (FSA4480) keeps DP and DN routed to the Audio DAC (Left/Right sound) instead of the USB controller.

- Audio Ground (AGND): The switch maintains ground separation to ensure high-current battery charging noise (1A-3A) does not bleed into the audio signal as buzzing or hum.

Here is a complete, clear, and brief explanation of each of these three Texas Instruments ICs and how they work together in mobile and USB-C audio/data designs:

1. TS5USBA2254 / TS3USBA225 (High-Speed USB & Negative-Rail Audio Switch)

- What it is: A specialized high-speed analog multiplexer/switch designed to route signals between a shared physical connector and internal chips.

- Core Functionality:

  - Dual Role Routing: Directs physical data pins (like USB D+/D-) to either the USB 2.0 PHY / SoC (for data transfer up to 480 Mbps) or to the Audio DAC (for analog audio output).

  - Negative Rail Capability: Can pass audio signals that swing below ground (down to -1.8V) without clipping or needing DC-blocking capacitors, ensuring clean audio without distortion.

  - Click-and-Pop Reduction: Integrates internal shunt resistors to eliminate audible clicks and pops when switching between modes or inserting a plug.

2. TS3A227E (Autonomous Audio Accessory Detection & Configuration Switch)

- What it is: An intelligent audio jack / headset management IC that detects what type of audio accessory is plugged in and automatically routes microphone and ground lines.

- Core Functionality:

  - Automatic Standard Detection (CTIA vs. OMTP): Standard 4-pole 3.5mm headsets have two competing layouts—CTIA (Ground on Sleeve, Mic on Ring2) and OMTP (Mic on Sleeve, Ground on Ring2). The TS3A227E detects the impedance, figures out the layout automatically, and routes the lines correctly without user intervention.

  - 3-Pole vs. 4-Pole Detection: Identifies whether the plug is a simple 3-pole headphone (no mic) or a 4-pole headset (with mic).

  - Key-Press Detection: Senses up to 4 inline headset button clicks (Play/Pause, Volume Up/Down, Voice Assistant) and informs the processor over I2C.

  - Ultra-low RON Ground FETs (60 mΩ): Provides a clean ground path to prevent crosstalk between audio channels.

3. TUSB32xD Family (e.g., TUSB320 / TUSB321 / TUSB322 – USB Type-C CC Logic & Port Controllers)

- What it is: A dedicated Type-C Configuration Channel (CC) controller that handles physical plug detection and role negotiation.

- Core Functionality:

  - Attach / Detach & Orientation Detection: Monitors the CC1 and CC2 lines to detect when a cable is inserted and which way it is oriented (normal or flipped).

  - Role Detection: Can configure the port as:

    - UFP / Sink: (e.g., phone acting as a device or charging).

    - DFP / Source: (e.g., phone powering an external accessory).

    - DRP (Dual Role Port): Switches automatically between source and sink roles.

  - Accessory Mode Detection: Detects when an audio adapter is attached (when both CC pins are pulled low via Ra) and signals the host processor to activate audio mode.

  - Current Advertisement: Detects power levels (Default 500mA/900mA, Medium 1.5A, or High 3.0A).

How They Work Together in a Phone

Plaintext

+------------------------------------------------------+

|                  USB Type-C Connector                |

+-----+------------------+-----------------------+-----+

|                  |                       |

VBUS / CC            D+/D-                 SBU1/SBU2

|                  |                       |

v                  v                       v

+---------------+   +---------------+       +---------------+

|   TUSB32xD    |   |  TS3USBA225   |       |   TS3A227E    |

| (CC Controller|   | (Audio / USB  |       | (MIC / GND    |

|  & Port Role) |   |  Data Switch) |       |  Cross-Mux)   |

+-------+-------+   +-------+-------+       +-------+-------+

|                   |                       |

+──────────────┐    |    ┌──────────────────+

v    v    v

+------------------+

|     Host SoC     |

| & Audio Codec/DAC|

+------------------+

- TUSB32xD monitors CC1/CC2 to detect whether a charger, data cable, or earphone is plugged in.

- TS3USBA225 switches the D+/D- lines to route either USB 2.0 digital data or analog stereo sound (L/R).

- TS3A227E auto-detects the headset wiring (CTIA vs. OMTP) and connects the microphone and ground paths cleanly without pops or buzzes.

Here is a concise summary of the entire end-to-end design and system flow:

1. The Core Problem & Solution

- Challenge: Smartphones need to handle High-Speed USB Data, Fast Charging (5V–20V), and Analog Stereo Audio/Mic through a single USB Type-C connector without interference.

- Solution: The USB-C standard uses Audio Adapter Accessory Mode, multiplexing existing pins (D+/D- for Left/Right audio, SBU1/SBU2 for Mic/Ground) while dedicating VBUS and GND for power delivery.

2. Detection & Mode Activation (The "Brain")

- Configuration Channel (CC Pins):

  - Charging: Only one CC pin changes voltage (Rd detected) → Phone activates the charging path and begins USB-PD negotiation.

  - Audio Plug: Both CC1 and CC2 pins drop to near ground (Ra detected) → Phone signals the host processor that an analog earphone/dongle is attached.

3. Signal Flow & Data Processing

- Sound Output: Digital audio → Audio DAC/Codec (converts to analog waves) → Analog Switch → D+/D- pins → Earphone speaker.

- Microphone Input: Sound pressure → Transducer/JFET (needs MICBIAS) → SBU pins → Analog Front-End (PGA) → Delta-Sigma ADC → DSP (Echo/Noise Cancellation) → OS Audio Pipeline (AudioFlinger/ALSA) → Apps.

4. Key Dedicated ICs and Their Roles

| IC | Primary Function |
| --- | --- |
| TUSB32xD | Type-C Port Controller: Senses CC1/CC2 pins, detects plug orientation, and flags whether a charger or audio adapter is connected. |
| TS3USBA225 / FSA4480 | High-Speed USB & Audio Switch: Multiplexes D+/D- lines between the SoC USB PHY (digital 1s/0s) and Audio DAC (L/R analog waves with negative voltage swing). |
| TS3A227E | Autonomous Audio Accessory Switch: Detects 3-pole vs. 4-pole headsets, auto-switches CTIA vs. OMTP wiring (Mic vs. Ground), and reads inline button presses. |
| PMIC / BQ-Series | Power Management: Safely regulates VBUS voltage to charge the battery. |